SUMMARY TABLE: GENERAL SCIENCE GRADE 10

Sub-Strand 3.1: Turning Effect of Force — Teacher Reference (Pre-filled)

| SUMMARY TABLE: GENERAL SCIENCE GRADE 10 |  |
| --- | --- |
| Sub-Strand | Sub-Strand 3.1: Turning Effect of Force |
| Driving Question | DRIVING QUESTION: Why does where you push, pull, or grip something determine how easy or hard it is to make it turn — and how do engineers and farmers in Kenya use this to design better tools? |

| INSTRUCTIONS |  |
| --- | --- |
| FOR TEACHERS: This is the teacher reference version — each row is pre-filled with expected student responses. Use it to assess student Summary Tables, identify gaps, and prepare discussion questions. The student version has blank cells for students to complete after each lesson. |

| Lesson # and Title | What did I observe? | What did I learn? | How does this explain the phenomenon? |
| --- | --- | --- | --- |
| Lesson 1 The Jembe Handle Puzzle — Launching the Phenomenon | Two learners — Amina and Juma — digging school-garden soil in Kericho using jembes with identical blade weights but different handle lengths. Amina (long handle ≈90 cm) finished quickly and easily; Juma (short handle ≈30 cm) struggled and tired rapidly. A ruler push-activity showed that pushing at the far end feels easier than pushing close to the pivot finger. | We have not yet learned the scientific explanation — we are still in the prediction stage. We noticed that handle length and grip position seem to matter. We generated predictions and questions. We created our first model sketch of force arrows and a pivot on a jembe. Our Driving Question Board is now open and holds all our current wonderings and questions. | Nothing has been formally explained yet — this is intentional. Our best current prediction is that gripping further from the metal blade makes digging easier, but we do not yet know WHY in scientific terms. We will investigate this in Lesson 2 using a metre rule and hanging masses to find the pattern. |
| Lesson 2 Moments of a Force at a Point — The Metre Rule Balancing Investigation | Learners balanced a metre rule on a pencil pivot using masses at measured distances on each side. They recorded weight (F in Newtons) and distance (d in cm/m) for at least five balanced configurations and computed F×d for both sides of the pivot in every trial. | For every balanced configuration, F₁×d₁ = F₂×d₂. This pattern is the Principle of Moments: a body is in rotational equilibrium when the sum of clockwise moments equals the sum of anticlockwise moments about the pivot. The moment of a force is M = F × d (units: N·m). | The Kericho jembe farmer's long handle works because the blade-soil contact point is the pivot. A longer handle increases the perpendicular distance d from the pivot to the line of effort. For the same effort force F, a larger d produces a larger moment M = F × d — meaning a greater turning effect on the blade — so the soil is broken with less physical effort from the farmer. |
| Lesson 3 Formalising the Moment Equation: M = F × d and the Principle of Moments | Learners watched an animation/simulation showing a seesaw with adjustable loads and pivot distances, and used digital devices or print media to research the definition, formula, and unit of the moment of a force. | The moment of a force M = F × d (unit: Newton-metre, Nm); the principle of moments (sum of clockwise moments = sum of anti-clockwise moments at equilibrium); the condition for rotational equilibrium; and that moving the jembe grip three times farther from the pivot (0.30 m to 0.90 m) produces a moment 3× greater for the same applied force. | The jembe handle puzzle: the long-handled jembe is easier to use not because the force applied is larger, but because the perpendicular distance d from the grip to the blade pivot is larger, producing a much greater moment M = F × d at the blade — making it easier to rotate the blade through hard soil with the same muscular effort. |
| Lesson 4 Anti-Parallel Forces and Couples: When Two Pushes Make a Turn | Learners observed a suspended metre rule rotate when two equal, opposite (anti-parallel) forces were applied at different positions along it — even though the net translational force was zero. They observed that moving the two forces further apart caused greater rotation, and reducing the separation reduced the rotation. | Learners learned that a COUPLE is a pair of equal, parallel forces acting in opposite directions (anti-parallel) whose lines of action do not coincide. The moment of a couple = F × d (where d is the perpendicular distance between the two forces). A couple produces pure rotation with zero net translational force. Real-life Kenyan examples include a matatu steering wheel, a pipe wrench used by a Kisumu plumber, and a tap or bottle cap turned with finger and thumb. | The jembe is NOT a couple: one effort force at the grip acts at a distance d from the blade pivot, so its moment is M = F × d and a longer handle gives a larger moment (Lessons 2 and 3). A couple needs two equal and opposite forces, as with two hands on a steering wheel: spreading the hands apart increases the distance d between the forces, so the same push from each hand gives a larger turning effect. Both ideas use the rule moment = force × distance, but the distance is measured differently: from the pivot to the force for a single force, and between the two forces for a couple. |
| Lesson 5 Calculating Moments of Anti-Parallel Forces — Numerical Problems | Learners solved three graded numerical problems (jembe, matatu door, Rift Valley ox-cart) using M = F × d, labelling each moment as clockwise or anticlockwise and calculating the net moment to determine the direction of rotation. | The turning effect (moment) of a force depends on BOTH the size of the force AND its perpendicular distance from the pivot. Each force has a moment about the pivot, and its direction (clockwise or anticlockwise) depends on which side of the pivot the force acts and which way it pushes. Moments in opposite senses are subtracted (as in the jembe and door problems); two opposite forces on opposite sides of the pivot, like the oxen on the cart crossbar, turn it the same way and their moments are added. The net moment decides which way the object rotates. This explains precisely — not just qualitatively — why a longer jembe handle makes digging easier. | When two anti-parallel forces act on an object, each creates a moment about the pivot: M = F × d. The two moments add if the forces act on opposite sides of the pivot (as with the oxen) and subtract if both forces act on the same side (as with the door). If the anticlockwise moments exceed the clockwise moments, the object rotates anticlockwise — and vice versa. The farmer in Kericho produces a much larger anticlockwise moment with a long handle (large d) than the soil resistance produces as a clockwise moment close to the blade pivot, so the blade digs in with relatively little effort. |
| Lesson 6 Walk-Around and Real-Life Application Mapping: Turning Effect of Force in Our Environment | Learners walked around the school compound and identified real-life examples of turning effect of force — including the school gate (pivot at hinge), water tap handle (pivot at spindle), window stay scissor hinge, flagpole halyard cleat, and a groundskeeper's crowbar. They also observed video clips of a construction crane at Mombasa port, long-handled pliers at Kamukunji market in Nairobi, and a boda-boda bicycle brake lever in Kisumu. In every case, effort was applied at a distance from a fixed pivot, producing a turning effect. | The turning effect of force (moment) is not unique to the jembe — it is the same principle operating in gates, taps, pliers, cranes, and bicycle brakes across Kenya. Moment = Force × perpendicular distance from pivot. A longer effort arm produces a greater moment for the same applied force, making turning easier. Engineers and tool-makers deliberately design long handles and lever arms to multiply the turning effect and reduce the effort needed by the user. | The Principle of Moments explains why the farmer in Kericho grips the jembe far from the blade: the greater the perpendicular distance from the pivot (the blade-soil contact point), the greater the moment produced by the same muscular effort. This same principle is exploited by Kenyan engineers designing crane booms, jua-kali artisans choosing long-handled pliers, and boda-boda manufacturers sizing brake levers — all are maximising the moment arm to make turning forces more effective with less effort. |
| Lesson 7 DQB Update and Model Building: Unifying the Turning Effect of Force | Learners observed their own Lesson 1 jembe sketches placed alongside all evidence collected across six lessons — metre-rule data tables, couple investigation notes, and walk-around photographs — revealing the gap between their initial naive model and a scientifically complete explanation of the turning effect of force. | Learners learned that a single equation, M = F x d, unifies every turning-effect phenomenon encountered in the unit: the jembe, the school gate, the seesaw, the bicycle brake, and the crane all involve a pivot, a force, and a perpendicular distance, and increasing the distance always produces a larger moment for the same applied force. | Learners explained why the farmer in Kericho finds the long-handled jembe easier by constructing a fully labelled scientific diagram showing that the effort moment of 72 Nm exceeds the load moment of 30 Nm while the short handle's 24 Nm does not, and by connecting this calculation to the updated Driving Question Board answer: where you push or grip something determines the moment arm, and the moment arm determines the turning effect produced by that force. |
| Lesson 8 Consolidation, Assessment and Reflection: The Final Explanation of Turning Effect of Force | Learners observed: (i) their own Lesson 1 prediction cards alongside their current understanding, making conceptual change visible; (ii) the metre-rule balancing task, where a physical system achieves rotational equilibrium only when masses are placed at distances that satisfy the Principle of Moments; (iii) the anchoring phenomenon image (jembe handle puzzle) displayed for the final time, now viewed through a fully developed scientific lens. | Learners demonstrated that: the moment of a force equals the force multiplied by its perpendicular distance from the pivot (M = F × d); a body is in rotational equilibrium when the sum of clockwise moments equals the sum of anti-clockwise moments; anti-parallel forces forming a couple produce a net turning effect equal to F × d with no resultant translational force; and the long-handled jembe is easier to use because the greater perpendicular distance from the pivot (blade-soil contact) produces a larger moment from the same applied force — fully explaining the anchoring phenomenon. | Learners constructed a three-sentence class Final Explanation: 'A moment is the turning effect of a force, calculated as the force multiplied by its perpendicular distance from the pivot. In the jembe, the blade-soil contact is the pivot — a longer handle increases the perpendicular distance, so the same effort from the farmer creates a much larger moment, making it easier to break hard soil. This is why engineers and farmers design tools with longer handles or wider grips: to maximise the turning effect without increasing the effort required.' |

ATTRIBUTION AND LICENSING

Curriculum Alignment: Structured in pedagogical alignment with the competence-based curriculum format and learning outcomes established by the Kenya Institute of Curriculum Development (KICD). This material is independently developed and is not officially endorsed or published by KICD.

Content & Pedagogy: Developed and adapted by the educator team at SeaVuria (registered 501(c)(3) in the United States and registered NGO in Kenya, No. 872-850A-BF11), who designed the pedagogical framework, lesson templates and exemplar lessons on which this material is based.

Production & Layout: Educational design and curriculum sequencing by SeaVuria. Technical layout, structural formatting, and digital optimization/implementation executed in partnership with ARES Education (registered 501(c)(3) in the United States and Canada (as Afretech Aid) and registered NGO in Kenya), with lesson drafting and compilation assisted by AI (Claude, Anthropic).

Licensing & Copyright: © 2026 SeaVuria and ARES. This work is licensed under a Creative Commons Attribution-NonCommercial 4.0 International License (CC BY-NC 4.0). To view a copy of this license, visit https://creativecommons.org/licenses/by-nc/4.0/. You are free to copy, redistribute, and adapt this material in any medium or format, provided appropriate credit is given and the material is not used for commercial purposes. KICD learning outcomes and curriculum designations reproduced here for alignment purposes remain the property of KICD and are not covered by this license. Linked external resources (e.g. CK-12, Khan Academy, PhET) are subject to their own licenses.