SUMMARY TABLE: PHYSICS GRADE 10

Sub-Strand 3.3: Introduction to Electronics — Teacher Reference (Pre-filled)

| SUMMARY TABLE: PHYSICS GRADE 10 |  |
| --- | --- |
| Sub-Strand | Sub-Strand 3.3: Introduction to Electronics |
| Driving Question | DRIVING QUESTION: Why do some materials conduct electricity while others do not, and how does this affect the safety and efficiency of household wiring and of devices such as solar panels, computers and MRI machines? |

| INSTRUCTIONS |  |
| --- | --- |
| FOR TEACHERS: This is the teacher reference version — each row is pre-filled with expected student responses. Use it to assess student Summary Tables, identify gaps, and prepare discussion questions. The student version has blank cells for students to complete after each lesson. |

| Lesson # and Title | What did I observe? | What did I learn? | How does this explain the phenomenon? |
| --- | --- | --- | --- |
| Lesson 1 Which Materials Light the Bulb? Launching the Phenomenon | We tested copper wire, glass, dry wood, plastic and a silicon diode in a 3 V circuit with a torch bulb. The bulb was bright with copper, and off with glass, dry wood and plastic. With the diode connected forward (forward-biased) the bulb lit, and with it reversed (reverse-biased) the bulb was off. So 1 of 4 ordinary materials conducts, and the diode conducts in one direction only. | Materials differ in whether they let current flow: copper is a conductor, while glass, wood and plastic are insulators that let almost no current through, which is why Mr Otieno can safely hold plastic-covered copper cable. The diode is different from both groups because it depends on direction. | We cannot yet explain why copper lets current flow and plastic does not, even though both are made of atoms with electrons, or why the diode works one way only. Our first class model shows electrons in each material, but it cannot explain these differences yet. Our DQB questions will guide what we investigate next. |
| Lesson 2 Classifying Conductors, Insulators and Semiconductors | We re-ran the test circuit (3 V, bulb): copper bright; glass, dry wood and plastic off; silicon diode lit forward and off in reverse. The resistivity card showed copper about 1.7 x 10^-8 ohm metre, germanium 0.46, silicon 2.3 x 10^3, dry wood about 10^8 to 10^11, glass about 10^10 to 10^14 and PVC about 10^13 or more. Silicon is about 1.4 x 10^11 times more resistive than copper. | Materials can be classified by resistivity: conductors (copper) have very low resistivity; semiconductors (germanium and silicon) are in between; insulators (wood, glass, PVC) have very high resistivity and allow almost no current. Conductivity is 1/rho. Household wiring uses copper for efficient current flow and PVC to keep current inside the wire so people can handle cables safely. | We can explain why Mr Otieno uses copper cable covered in PVC for a Nairobi house at 240 V: copper's low resistivity carries current efficiently and PVC's very high resistivity almost stops current reaching a person. We cannot yet explain WHY the resistivities differ so much or why silicon conducts a little, which we will investigate next. |
| Lesson 3 Intrinsic Semiconductors: Silicon and Germanium | We observed that the silicon diode (1N4007) forward-connected lit the 3 V bulb, as in Lesson 1, and that the data show silicon (rho about 2.3 x 10^3 ohm metre) and germanium (about 0.46 ohm metre) are far more resistive than copper (1.7 x 10^-8 ohm metre) but far less than glass. The temperature dataset showed the resistivity of pure silicon falling as it was warmed, while copper's resistance rose slightly. | We learned that pure (intrinsic) silicon and germanium have a small energy gap (1.1 eV and 0.67 eV), so at room temperature a few electrons gain enough energy to become free electrons, each leaving a hole to make an electron-hole pair. Germanium, with the smaller gap, conducts more than silicon. Warming creates more pairs so conductivity rises. | We explained that silicon conducts a little because its small gap lets a few electrons cross, unlike glass, wood and PVC with a gap above about 5 eV that allow almost no free electrons, and unlike copper, where the bands overlap. This helps explain why semiconductors sit between copper wire and its plastic cover in Mr Otieno's Nairobi house, and raises the question of how to control their conduction, which we will investigate next. |
| Lesson 4 Doping: Making n-type and p-type Semiconductors | We re-tested the 1N4007 diode: the bulb lit when it was forward-connected (forward-biased) and stayed off when it was reverse-connected (reverse-biased). Using counters, we saw that replacing one silicon atom with phosphorus (5 outer electrons) leaves an extra free electron, and replacing one with boron (3 outer electrons) leaves a hole. The data card showed that doping about 1 atom in 10^7 to 10^8 raises the conductivity of silicon by a factor of many thousands. | Doping makes extrinsic semiconductors from pure (intrinsic) silicon. Phosphorus gives n-type (majority carriers: electrons) and boron gives p-type (majority carriers: holes). Doped silicon is still electrically neutral overall. Doping lets us control conductivity, and joining p-type and n-type makes a p-n junction, which is the diode. | Pure silicon conducts very poorly (resistivity about 2.3 x 10^3 ohm metre), but a tiny amount of phosphorus or boron adds free electrons or holes, so conductivity rises thousands of times in a controlled way. A p-n junction lets current pass when forward-biased (bulb lit) and blocks it when reverse-biased (bulb off), which is why doped silicon is the basis of diodes, solar panels like those at Garissa, and microchips in phones and computers. |
| Lesson 5 PhET Simulation: Testing the Band Model with Digital Evidence | In the simulation we changed one variable at a time. Gap: a metal (0 eV) gave a high current reading, germanium (0.67 eV) and silicon (1.1 eV) gave a few free electrons, and an insulator (above about 5 eV) gave almost none. Warming silicon freed more electrons and holes. Adding phosphorus gave extra free electrons (n-type) and adding boron gave holes (p-type), and the doped silicon stayed electrically neutral overall. | The simulation supported the band model: the size of the energy gap controls how many electrons can move, silicon and germanium conduct a little and more when warmer, and doping raises conduction in a controlled way. We changed our model to show the four gaps side by side, more free electrons and holes at higher temperature, and neutral doped silicon. A simulation simplifies real atoms and cannot show the bulb or the 3 V cells, so it supports a model but does not replace the bulb test. | Copper lights the bulb because its bands overlap (gap 0 eV), so electrons move easily. Glass, wood and plastic almost do not, because the gap is above about 5 eV and almost no electrons can move, which is why Mr Otieno can safely hold PVC-covered copper cable. Silicon sits in between and conducts more when warmer, and phosphorus (n-type) or boron (p-type) change its conduction in a controlled way. |
| Lesson 6 Superconductors and MRI Magnets: When Resistance Drops to Zero | We observed that a YBCO disc at room temperature did not levitate a magnet, but cooled to 77 K (-196 C) in liquid nitrogen it made the magnet float. We compared Tc values: mercury 4.2 K, NbTi 9.2 K, YBCO about 92 K, and noted which coolant (77 K or 4.2 K) is cold enough for each. | We learned that a superconductor has zero resistance below its critical temperature (Tc), unlike copper, which always has some resistance (1.7 x 10^-8 ohm metre at about 20 C). With zero resistance a large current flows with no energy lost as heat. | We explain that Dr Wekesa's MRI machine uses NbTi coils cooled to 4.2 K with liquid helium, below the Tc of 9.2 K, so the magnet carries a large current without energy loss and gives a field of about 1.5 T to 3 T. Household wiring still uses copper and PVC because room temperature is far above every Tc we studied. |
| Lesson 7 Case Studies and the Final Answer to the Driving Question | What we observed: copper lit the bulb brightly; glass, dry wood and plastic did not; the silicon diode (1N4007) lit the bulb only when forward-connected. In the Kenyan cases we saw silicon p-n junctions at work in the Garissa solar plant (50 MW) and in microchips, and an NbTi superconducting magnet cooled to 4.2 K in the Kenyatta National Hospital MRI, with a YBCO disc levitating a magnet at 77 K. | What we learned: materials differ in resistivity (copper 1.7 x 10^-8 ohm metre up to PVC about 10^13 or more) and in energy gap (metals 0 eV, germanium 0.67 eV, silicon 1.1 eV, insulators above about 5 eV). Doping silicon with phosphorus (n-type) or boron (p-type) raises conductivity in a controlled way, and a p-n junction conducts one way only. Superconductors have zero resistance below their critical temperature. | How we explained it: Copper conducts because its outer electrons are free to move (bands overlap), while glass, wood and plastic have a large gap so almost no electrons can move, making PVC a safe cover for Mr Otieno's wires. Silicon and germanium conduct a little, more when warmer, and doping with phosphorus or boron greatly raises conductivity, so p-n junctions make solar panels and microchips work. Below Tc, NbTi has zero resistance so MRI magnets carry large current without energy loss, which answers our Driving Question. |

ATTRIBUTION AND LICENSING

Curriculum Alignment: Structured in pedagogical alignment with the competence-based curriculum format and learning outcomes established by the Kenya Institute of Curriculum Development (KICD). This material is independently developed and is not officially endorsed or published by KICD.

Content & Pedagogy: Developed and adapted by the educator team at SeaVuria (registered 501(c)(3) in the United States and registered NGO in Kenya, No. 872-850A-BF11), who designed the pedagogical framework, lesson templates and exemplar lessons on which this material is based.

Production & Layout: Educational design and curriculum sequencing by SeaVuria. Technical layout, structural formatting, and digital optimization/implementation executed in partnership with ARES Education (registered 501(c)(3) in the United States and Canada (as Afretech Aid) and registered NGO in Kenya), with lesson drafting and compilation assisted by AI (Claude, Anthropic).

Licensing & Copyright: © 2026 SeaVuria and ARES. This work is licensed under a Creative Commons Attribution-NonCommercial 4.0 International License (CC BY-NC 4.0). To view a copy of this license, visit https://creativecommons.org/licenses/by-nc/4.0/. You are free to copy, redistribute, and adapt this material in any medium or format, provided appropriate credit is given and the material is not used for commercial purposes. KICD learning outcomes and curriculum designations reproduced here for alignment purposes remain the property of KICD and are not covered by this license. Linked external resources (e.g. CK-12, Khan Academy, PhET) are subject to their own licenses.