# Grade 10 Physics: Mechanical Properties of Materials

## Competency-Based Education (CBE) Lesson Sequence

## SECTION A: SUB-STRAND OVERVIEW

### Sub-Strand: 1.3 Mechanical Properties of Materials

Strand: 1.0 Mechanics and Thermal Physics
Grade Level: 10
Total Duration: 12 periods (6 lessons × 2 periods each)

### 1. Phenomenon and Driving Question

Phenomenon:
Multiple interconnected phenomena anchor this unit: - Springs and rubber bands stretch when weights are hung from them, but return to their original length when the weight is removed - Clay can be molded and shaped permanently, while glass shatters when similar forces are applied - Different materials (metals, plastics, wood, rubber) behave very differently under the same load—some bend, some stretch, some break - Matatu shock absorbers compress and extend repeatedly to provide smooth rides on rough Kenyan roads - Construction materials like steel reinforcement bars, concrete, and timber must support heavy loads without failing

Main Driving Question:
“How do materials change shape when forces are applied, and why do some return to their original shape while others don’t?”

Supporting Questions: - Why does a rubber band snap back but clay stays molded? - How can we measure and compare the strength and flexibility of different materials? - Why do some materials bend or stretch while others break? - How do engineers choose the right materials for bridges, buildings, and vehicles? - What makes a material suitable for specific applications in Kenya’s construction and manufacturing sectors?

### 2. Learning Outcomes

By the end of this sub-strand, learners should be able to:

Knowledge and Understanding: - Explain mechanical properties: elasticity, plasticity, ductility, malleability, brittleness, hardness, stiffness, strength, and toughness - Describe Hooke’s Law and the relationship F = kx between force and extension - Define stress (σ = F/A), strain (ε = ΔL/L₀), and Young’s modulus (E = σ/ε) - Interpret stress-strain graphs to identify elastic limit, plastic deformation, and breaking point - Apply knowledge of material properties to real-world contexts in Kenya

Skills: - Conduct experiments to verify Hooke’s Law and determine spring constants - Measure and calculate stress, strain, and Young’s modulus from experimental data - Analyze stress-strain graphs to compare material behaviors - Select appropriate materials for specific applications based on their mechanical properties - Use digital tools (PhET simulations, spreadsheets) to model material behavior - Collaborate effectively in laboratory investigations

Attitudes and Values: - Responsibility: Handle laboratory equipment safely and report accurate data with integrity - Unity: Work collaboratively in teams, respecting diverse perspectives - Excellence: Strive for precision in measurements and calculations - Integrity: Record honest experimental results, even when unexpected - Appreciate the importance of material selection for safety and sustainability

### 3. Key Concepts and Content

Material Properties: - Elasticity: Ability to return to original shape after deformation (rubber bands, springs) - Plasticity: Permanent deformation without breaking (clay, aluminum foil) - Ductility: Ability to be drawn into wires (copper, gold) - Malleability: Ability to be hammered into sheets (aluminum, lead) - Brittleness: Tendency to break without significant deformation (glass, ceramics) - Hardness: Resistance to scratching or indentation (diamond, steel) - Stiffness: Resistance to deformation (steel vs rubber) - Strength: Ability to withstand applied forces without breaking - Toughness: Ability to absorb energy before breaking

Hooke’s Law: - Formula: F = kx (Force = spring constant × extension) - Proportionality between force and extension in elastic region - Spring constant (k) as a measure of stiffness (units: N/m) - Elastic limit: maximum extension before permanent deformation - Applications: springs, shock absorbers, measuring instruments

Stress and Strain: - Stress (σ): Force per unit cross-sectional area - Formula: σ = F/A - Units: Pa (Pascals) or N/m² - Types: tensile, compressive, shear - Strain (ε): Fractional change in length - Formula: ε = ΔL/L₀ (change in length / original length) - Dimensionless quantity (no units) - Expressed as decimal or percentage

Young’s Modulus: - Formula: E = σ/ε (stress / strain) - Units: Pa or N/m² - Measure of material stiffness - High E = stiff material (steel, concrete) - Low E = flexible material (rubber, plastic)

Stress-Strain Graphs: - Elastic region: Linear relationship, material returns to original shape - Elastic limit (yield point): Maximum stress before permanent deformation - Plastic region: Permanent deformation occurs - Ultimate tensile strength: Maximum stress material can withstand - Breaking point: Material fractures - Area under curve: Energy absorbed (toughness)

Applications: - Construction: Steel reinforcement bars, concrete, timber selection - Transportation: Matatu shock absorbers, tire materials, vehicle frames - Manufacturing: Jua Kali metalworking, wire drawing, sheet metal forming - Safety equipment: Helmets, protective gear, safety cables - Everyday items: Springs, rubber bands, cables, ropes

### 4. Core Competencies Addressed

- Critical Thinking and Problem Solving: Analyzing material behavior, interpreting graphs, selecting appropriate materials

- Collaboration and Communication: Group experiments, Think-Pair-Share discussions, presenting findings

- Creativity and Imagination: Designing investigations, proposing material applications

- Digital Literacy: Using PhET simulations, Khan Academy videos, spreadsheets for data analysis

- Learning to Learn: Building and revising models, tracking understanding on DQB

- Citizenship: Applying knowledge to local contexts (construction safety, resource management)

### 5. Values Integration

- Responsibility: Safe handling of equipment, accurate data recording, proper disposal of materials

- Unity: Teamwork in laboratory investigations, respecting diverse ideas

- Excellence: Precision in measurements, thorough analysis, quality work

- Integrity: Honest reporting of results, acknowledging limitations

- Respect: Valuing contributions of all team members, listening actively

- Patriotism: Applying knowledge to improve Kenya’s construction and manufacturing sectors

### 6. Pertinent and Contemporary Issues (PCIs)

- Safety and Health: Laboratory safety, construction site safety, material failure prevention

- Industrialization: Material selection in manufacturing, Jua Kali sector innovation

- Environmental Conservation: Sustainable material use, recycling, reducing waste

- Resource Management: Efficient use of materials, cost-effectiveness in construction

- Infrastructure Development: Building safe structures, road construction, bridge engineering

- Economic Development: Supporting local industries through informed material choices

### 7. Links to Other Subjects

- Mathematics: Calculations, graphing, proportional relationships, unit conversions

- Chemistry: Atomic structure and bonding affecting material properties

- Geography: Natural resources, mining, material availability in Kenya

- Business Studies: Cost analysis, material procurement, quality control

- Design and Technology: Material selection for projects, prototyping

- Agriculture: Tools and equipment materials, storage structures

### 8. Assessment Overview

Formative Assessment (Ongoing): - Initial models of material behavior (Lesson 1) - POE predictions and explanations (all lessons) - Driving Question Board contributions (all lessons) - Laboratory investigation observations and data (Lessons 2-5) - Calculations and problem-solving (Lessons 3-4) - Graph interpretation (Lesson 5) - Model revisions (Lessons 2, 3, 5) - Exit tickets (all lessons) - Think-Pair-Share participation (all lessons)

Summative Assessment (Lesson 6): - Final Explanatory Model: Comprehensive diagram/explanation answering the driving question - Written Assessment: Multiple choice, calculations, graph interpretation, short answer - Practical Demonstration: Material testing and analysis - Application Task: Material selection for a Kenya-specific scenario (construction, manufacturing, or transportation)

## SECTION B: DETAILED LESSON PLANS

## LESSON 1: Exploring Material Properties and Initial Models

Duration: 2 periods (80 minutes)
Focus: Phenomenon introduction, prior knowledge activation, initial models, material properties

### Learning Outcomes:

- Observe and describe how different materials respond to applied forces

- Activate prior knowledge about materials and forces

- Create initial explanatory models for material behavior

- Define and classify material properties: ductility, malleability, elasticity, brittleness, hardness, stiffness, strength, toughness

### Materials and Resources:

- Various materials: springs, rubber bands, clay, glass rod (demonstration), wire, plastic, wood, metal strips

- Weights or masses (100g, 200g, 500g)

- Rulers, measuring tapes

- Safety goggles

- Driving Question Board (poster/digital)

- Initial Model template (paper/digital)

- Digital Resources:

  - Video: “Material Properties Introduction” Here

  - PhET Simulation: “Hooke’s Law” Here

  - Khan Academy: “Introduction to Materials Science” Video  Reading

### Lesson Flow:

Period 1 (40 minutes): Phenomenon and Prior Knowledge

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-5 min | Phenomenon demonstration: Stretch spring, mold clay, show glass breaking (video) | Engage learners, create cognitive dissonance |
| 5-10 min | Present Driving Question: “How do materials change shape when forces are applied, and why do some return to their original shape while others don’t?” | Focus inquiry |
| 10-15 min | Individual brainstorm: What do you already know about materials and forces? | Activate prior knowledge |
| 15-25 min | Think-Pair-Share: Discuss prior knowledge, share with class | Build on existing understanding |
| 25-35 min | Hands-on exploration: Learners test 5-6 different materials with forces (stretch, bend, compress) | Generate observations |
| 35-40 min | Record observations: What happened to each material? | Document evidence |

Period 2 (40 minutes): Initial Models and Material Properties

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-10 min | Individual initial model: Draw and explain why materials behave differently | Externalize thinking |
| 10-15 min | Share models in pairs, discuss similarities/differences | Peer learning |
| 15-25 min | Direct instruction: Material properties (elasticity, plasticity, ductility, malleability, brittleness, hardness, stiffness, strength, toughness) with examples | Build vocabulary and concepts |
| 25-35 min | Classification activity: Sort materials by properties using hands-on testing | Apply new knowledge |
| 35-38 min | Start Driving Question Board: What do we know? What do we need to know? | Track learning journey |
| 38-40 min | Exit ticket: Define two material properties and give examples | Check understanding |

### Formative Assessment:

- Observation of hands-on exploration

- Initial models (collect and review)

- Classification activity accuracy

- DQB contributions

- Exit ticket responses

### Differentiation:

- Support: Provide vocabulary cards with definitions and images, pre-labeled material samples

- Extension: Challenge learners to predict which properties are most important for specific applications (bridge cables, cooking pots, phone cases)

- ELL: Visual demonstrations, bilingual vocabulary lists, peer support

### Safety Considerations:

- Wear safety goggles during demonstrations

- Handle glass carefully (teacher demonstration only)

- Supervise use of weights to prevent dropping on feet

- Clear workspace of tripping hazards

## LESSON 2: Investigating Hooke’s Law

Duration: 2 periods (80 minutes)
Focus: F = kx, spring constant, proportionality, elastic limit, POE cycle

### Learning Outcomes:

- Conduct an investigation to verify Hooke’s Law (F = kx)

- Calculate spring constant (k) from experimental data

- Identify the elastic limit of a spring

- Interpret graphs of force vs. extension

- Revise initial models based on new evidence

### Materials and Resources:

- Springs (various stiffnesses)

- Masses (50g, 100g, 200g, 500g sets)

- Retort stands and clamps

- Rulers or meter sticks

- Graph paper or spreadsheet software

- Calculators

- Digital Resources:

  - PhET Simulation: “Hooke’s Law” (data collection mode)

  - Video: “Hooke’s Law Explained” (Khan Academy)

  - Graphing tool: Desmos or Excel

### Lesson Flow:

Period 1 (40 minutes): POE and Investigation

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-5 min | Review Lesson 1: What is elasticity? What happens when you stretch a spring? | Connect to prior learning |
| 5-10 min | PREDICT: What will happen to spring extension as we add more weight? Draw prediction graph | Engage prior knowledge |
| 10-15 min | OBSERVE: Demonstration of spring extension with increasing masses | Provide evidence |
| 15-20 min | Group investigation setup: Hang spring, measure original length, add masses incrementally | Hands-on data collection |
| 20-35 min | Collect data: Record mass and total extension for 5-6 different masses | Generate evidence |
| 35-40 min | EXPLAIN: Does your data match your prediction? Why or why not? | Confront misconceptions |

Period 2 (40 minutes): Analysis and Model Revision

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-10 min | Plot force (F = mg) vs. extension (x) graph | Visualize relationship |
| 10-15 min | Analyze graph: Is it linear? What does the slope represent? | Identify patterns |
| 15-25 min | Introduce Hooke’s Law: F = kx, where k is spring constant | Formalize understanding |
| 25-30 min | Calculate spring constant from graph slope or data | Apply mathematics |
| 30-35 min | Discuss elastic limit: What happens if we stretch too far? | Extend understanding |
| 35-38 min | Revise initial model: Add Hooke’s Law and elastic limit | Update thinking |
| 38-40 min | Exit ticket: Explain Hooke’s Law in your own words | Check understanding |

### Formative Assessment:

- POE predictions and explanations

- Data collection accuracy and organization

- Graph construction and interpretation

- Spring constant calculations

- Revised models (collect and review)

- Exit ticket responses

### Differentiation:

- Support: Provide pre-made data tables, graph templates, step-by-step calculation guides

- Extension: Investigate multiple springs with different stiffnesses, compare k values, explore energy stored in springs

- ELL: Visual instructions, formula cards, peer support for technical vocabulary

### Safety Considerations:

- Secure retort stands to prevent tipping

- Use appropriate masses to avoid over-stretching springs

- Keep feet clear of hanging masses

- Wear safety goggles

## LESSON 3: Understanding Stress and Strain

Duration: 2 periods (80 minutes)
Focus: σ = F/A, ε = ΔL/L₀, calculations, units, material comparison

### Learning Outcomes:

- Define stress (σ) as force per unit area

- Define strain (ε) as fractional change in length

- Calculate stress and strain from experimental or given data

- Explain why stress and strain are more useful than force and extension for comparing materials

- Revise models to include stress-strain relationships

### Materials and Resources:

- Wire samples (different materials: copper, steel, aluminum)

- Masses and weight hangers

- Vernier calipers or micrometers (for measuring wire diameter)

- Meter sticks or measuring tapes

- Calculators

- Digital Resources:

  - Video: “Stress and Strain Explained” (Khan Academy)

  - Interactive simulation: “Stress-Strain Relationships”

  - Spreadsheet template for calculations

### Lesson Flow:

Period 1 (40 minutes): Introducing Stress and Strain

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-5 min | Review Hooke’s Law: F = kx. Ask: How can we compare different materials fairly? | Create need for new concepts |
| 5-10 min | Demonstration: Two wires (thick and thin) with same force—different extensions | Show limitation of F and x alone |
| 10-20 min | Direct instruction: Stress (σ = F/A) and strain (ε = ΔL/L₀), units, meaning | Introduce new concepts |
| 20-25 min | Think-Pair-Share: Why is stress more useful than force? Why is strain more useful than extension? | Deepen understanding |
| 25-35 min | Worked examples: Calculate stress and strain from given data | Model problem-solving |
| 35-40 min | Practice problems: Learners calculate stress and strain individually | Apply new skills |

Period 2 (40 minutes): Investigation and Application

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-5 min | Review calculations, address common errors | Clarify misconceptions |
| 5-20 min | Group investigation: Measure wire diameter, original length, apply force, measure extension | Collect real data |
| 20-30 min | Calculate stress and strain for your wire sample | Apply formulas to real data |
| 30-35 min | Compare results across groups (different materials): Which material has higher stress for same strain? | Analyze patterns |
| 35-38 min | Revise model: Add stress and strain concepts | Update understanding |
| 38-40 min | Exit ticket: Calculate stress or strain from given data | Check mastery |

### Formative Assessment:

- Think-Pair-Share discussions

- Practice problem accuracy

- Investigation data collection and calculations

- Group comparisons and analysis

- Revised models

- Exit ticket calculations

### Differentiation:

- Support: Provide formula cards, step-by-step calculation guides, pre-measured wire dimensions

- Extension: Explore different types of stress (tensile, compressive, shear), calculate percentage strain

- ELL: Visual formula representations, worked example cards, bilingual glossaries

### Safety Considerations:

- Handle wire carefully to avoid cuts

- Secure apparatus to prevent falling masses

- Wear safety goggles

- Supervise use of vernier calipers/micrometers

## LESSON 4: Exploring Young’s Modulus

Duration: 2 periods (80 minutes)
Focus: E = σ/ε, material stiffness comparison, applications

### Learning Outcomes:

- Define Young’s modulus (E) as the ratio of stress to strain

- Calculate Young’s modulus from experimental or given data

- Compare materials based on their Young’s modulus values

- Explain the difference between stiffness and strength

- Apply knowledge to select materials for specific applications

### Materials and Resources:

- Data from Lesson 3 (stress and strain values)

- Reference table of Young’s modulus values for common materials

- Calculators

- Sample materials for comparison (steel, aluminum, rubber, plastic)

- Digital Resources:

  - Video: “Young’s Modulus Explained” (YouTube)

  - Interactive table: “Material Properties Database”

  - PhET Simulation: “Elasticity” (advanced mode)

### Lesson Flow:

Period 1 (40 minutes): Understanding Young’s Modulus

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-5 min | Review stress and strain: What do they tell us about materials? | Connect to prior learning |
| 5-10 min | Pose question: How can we measure how “stiff” a material is? | Create cognitive need |
| 10-20 min | Direct instruction: Young’s modulus (E = σ/ε), units (Pa), meaning (stiffness) | Introduce concept |
| 20-25 min | Worked examples: Calculate E from stress-strain data | Model calculations |
| 25-35 min | Practice problems: Learners calculate Young’s modulus | Apply new skill |
| 35-40 min | Think-Pair-Share: What does a high E value tell us? Low E value? | Deepen understanding |

Period 2 (40 minutes): Material Comparison and Applications

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-10 min | Calculate Young’s modulus from Lesson 3 data | Apply to real data |
| 10-15 min | Compare calculated values with reference table | Validate results |
| 15-25 min | Group activity: Given scenarios (bridge cable, shock absorber, building beam, phone case), select best material based on E values | Apply knowledge |
| 25-30 min | Groups present material choices and justifications | Share reasoning |
| 30-35 min | Discuss stiffness vs. strength: Can a material be stiff but weak? Strong but flexible? | Clarify distinctions |
| 35-38 min | Update DQB: What have we learned? What questions remain? | Track progress |
| 38-40 min | Exit ticket: Explain Young’s modulus and why it’s useful | Check understanding |

### Formative Assessment:

- Practice problem accuracy

- Young’s modulus calculations from real data

- Material selection justifications

- Group presentations

- DQB contributions

- Exit ticket explanations

### Differentiation:

- Support: Provide formula cards, calculator support, simplified scenarios

- Extension: Research Young’s modulus of advanced materials (carbon fiber, composites), explore temperature effects

- ELL: Visual scenario cards, bilingual material property tables, peer support

### Safety Considerations:

- No major safety concerns (primarily calculation-based lesson)

- Handle sample materials carefully

## LESSON 5: Analyzing Stress-Strain Graphs

Duration: 2 periods (80 minutes)
Focus: Graph interpretation, elastic/plastic regions, elastic limit, breaking point, energy storage

### Learning Outcomes:

- Interpret stress-strain graphs to identify key features: elastic region, elastic limit, plastic region, ultimate tensile strength, breaking point

- Distinguish between elastic and plastic deformation

- Explain what the area under a stress-strain curve represents (energy absorbed)

- Compare materials using stress-strain graphs

- Revise models to include complete understanding of material behavior

### Materials and Resources:

- Stress-strain graphs for various materials (steel, aluminum, rubber, glass, copper)

- Graph analysis worksheets

- Colored pencils for annotating graphs

- Digital Resources:

  - Video: “Understanding Stress-Strain Curves” (YouTube)

  - Interactive simulation: “Stress-Strain Graph Explorer”

  - Khan Academy: “Elastic and Plastic Deformation”

### Lesson Flow:

Period 1 (40 minutes): Graph Features and Interpretation

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-5 min | Review: What are stress and strain? What is Young’s modulus? | Activate prior knowledge |
| 5-10 min | Display stress-strain graph for steel: What do you notice? | Generate observations |
| 10-25 min | Direct instruction: Key features of stress-strain graphs (elastic region, elastic limit/yield point, plastic region, ultimate tensile strength, breaking point, area under curve) | Build interpretation skills |
| 25-35 min | Guided practice: Annotate a stress-strain graph, identify all key features | Apply new knowledge |
| 35-40 min | Think-Pair-Share: What’s the difference between elastic and plastic deformation? Give examples | Clarify concepts |

Period 2 (40 minutes): Material Comparison and Applications

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-10 min | Compare stress-strain graphs for different materials (steel, rubber, glass, aluminum) | Analyze patterns |
| 10-15 min | Group discussion: Which material is stiffest? Strongest? Toughest? Most brittle? | Apply graph interpretation |
| 15-25 min | Application scenarios: Match materials to uses based on graph features (construction beam = high strength, shock absorber = large elastic region, safety glass = high toughness) | Connect to real world |
| 25-30 min | Discuss energy storage: Why does area under curve matter? (Shock absorbers, safety equipment) | Deepen understanding |
| 30-35 min | Revise model: Create comprehensive explanation of material behavior including all concepts learned | Synthesize learning |
| 35-38 min | Update DQB: Can we answer the driving question now? | Prepare for synthesis |
| 38-40 min | Exit ticket: Sketch and label a stress-strain graph | Check mastery |

### Formative Assessment:

- Graph annotation accuracy

- Think-Pair-Share discussions

- Material comparison analysis

- Application scenario justifications

- Revised models (collect and review)

- Exit ticket graphs

### Differentiation:

- Support: Provide partially labeled graphs, color-coded regions, simplified comparison tasks

- Extension: Calculate energy absorbed from graph area, research advanced materials (shape-memory alloys, composites)

- ELL: Visual graph guides, bilingual labels, peer support

### Safety Considerations:

- No major safety concerns (primarily analysis-based lesson)

## LESSON 6: Applications, Synthesis, and Assessment

Duration: 2 periods (80 minutes)
Focus: Kenya contexts (construction, matatus, Jua Kali), final explanation, DQB completion, summative assessment

### Learning Outcomes:

- Apply knowledge of mechanical properties to real-world Kenya contexts

- Create a comprehensive final explanatory model answering the driving question

- Complete the Driving Question Board

- Demonstrate mastery through summative assessment

- Reflect on learning journey

### Materials and Resources:

- Case study materials: Kenya construction projects, matatu shock absorbers, Jua Kali metalworking

- Final model templates

- Assessment materials (written test, practical task, rubrics)

- Digital Resources:

  - Video: “Material Science in Construction” (Kenya context)

  - Case studies: “Jua Kali Innovation,” “Building Safe Structures in Kenya”

  - Virtual tour: Construction site or manufacturing facility

### Lesson Flow:

Period 1 (40 minutes): Applications and Final Model

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-5 min | Review unit: What have we learned about materials and forces? | Activate all prior learning |
| 5-15 min | Case study exploration: Groups examine Kenya-specific applications (Group 1: Building construction and steel reinforcement, Group 2: Matatu shock absorbers and suspension, Group 3: Jua Kali metalworking and material selection, Group 4: Bridge construction and cable materials) | Apply to authentic contexts |
| 15-25 min | Groups analyze: What material properties are most important? Why? What would happen if wrong materials were used? | Critical thinking |
| 25-30 min | Groups present findings | Share applications |
| 30-38 min | Individual final model: Create comprehensive diagram/explanation answering “How do materials change shape when forces are applied, and why do some return to their original shape while others don’t?” Include all concepts: properties, Hooke’s Law, stress, strain, Young’s modulus, stress-strain graphs | Synthesize learning |
| 38-40 min | Complete DQB: Answer driving question as a class | Celebrate learning |

Period 2 (40 minutes): Summative Assessment

| Time | Activity | Purpose |
| --- | --- | --- |
| 0-30 min | Written assessment: Multiple choice (10 questions), calculations (stress, strain, Young’s modulus, Hooke’s Law - 4 problems), graph interpretation (2 questions), short answer (2 questions on applications) | Assess knowledge and skills |
| 30-38 min | Practical task: Given material samples and a scenario, select and justify best material choice | Assess application |
| 38-40 min | Reflection: What was most interesting? Most challenging? How will you use this knowledge? | Metacognition |

### Summative Assessment Components:

1. Final Explanatory Model (30 points) - Rubric criteria: - Addresses driving question completely (5 points) - Includes all key concepts (properties, Hooke’s Law, stress, strain, Young’s modulus, graphs) (10 points) - Uses accurate scientific terminology (5 points) - Provides clear explanations with examples (5 points) - Visual representation is clear and labeled (5 points)

2. Written Assessment (40 points) - Multiple choice: 10 questions × 1 point = 10 points - Calculations: 4 problems × 5 points = 20 points - Graph interpretation: 2 questions × 3 points = 6 points - Short answer: 2 questions × 2 points = 4 points

3. Practical Task (20 points) - Material selection appropriateness (10 points) - Justification using mechanical properties (5 points) - Consideration of Kenya context (safety, cost, availability) (5 points)

4. Participation and Process (10 points) - DQB contributions throughout unit (3 points) - Laboratory investigation participation (3 points) - Model revisions showing growth (4 points)

Total: 100 points

### Differentiation:

- Support: Extended time, simplified scenarios, formula sheets, calculator support

- Extension: Design challenge (design a structure or device using optimal materials), research emerging materials

- ELL: Bilingual assessments, visual supports, oral explanation option

### Safety Considerations:

- No major safety concerns (assessment-based lesson)

- Supervise practical task material handling

## SECTION C: LESSON SUMMARY TABLE

This table provides a minute-by-minute overview of the learner experience across all 6 lessons (12 periods).

### LESSON 1: Exploring Material Properties and Initial Models (2 periods)

| Learner Experience | Resource Link | Teacher Moves | Sensemaking Strategy | Formative Assessment |
| --- | --- | --- | --- | --- |
| Period 1, Min 0-5: Watch teacher demonstrate stretching spring, molding clay, and video of glass breaking. Notice different behaviors. | Video: “Material Breaking Compilation” (YouTube) | Perform demonstrations safely. Ask: “What did you notice? What surprised you?” Avoid explaining yet. | Phenomenon-based engagement: Learners observe puzzling differences in material behavior, creating need to explain. | Observe facial expressions and initial reactions. Listen for spontaneous questions. |
| Period 1, Min 5-10: Listen to Driving Question: “How do materials change shape when forces are applied, and why do some return to their original shape while others don’t?” Write question in notebook. | Driving Question Board (poster/digital display) | Display DQB prominently. Read question aloud. Ask: “What do you think? Turn and talk to a partner.” | Question-driven inquiry: Driving question focuses entire unit investigation. | Note initial hypotheses shared in pairs. |
| Period 1, Min 10-15: Individual brainstorm: Write/draw everything you know about materials and forces. | Brainstorm template (paper/digital) | Circulate, encourage all ideas without judgment. Prompt: “Think about materials you use every day. What happens when you push, pull, or bend them?” | Prior knowledge activation: Learners access existing schemas to build on. | Review brainstorms to identify prior knowledge and misconceptions. |
| Period 1, Min 15-25: Think-Pair-Share: Share brainstorm with partner, discuss similarities/differences. Share key ideas with whole class. Teacher records on board. | Chart paper or digital board for recording | Facilitate sharing. Ask: “What patterns do you notice? What do many of you already know?” Record ideas without correcting. | Collaborative knowledge building: Learners learn from peers, teacher maps collective understanding. | Listen for common ideas and misconceptions to address later. |
| Period 1, Min 25-35: Hands-on exploration at stations: Test 5-6 materials (spring, rubber band, clay, wire, plastic strip, wood) by stretching, bending, compressing. Record what happens. | Materials: springs, rubber bands, clay, wire, plastic, wood, weights, rulers | Set up stations with clear instructions. Circulate, ask: “What happens when you apply force? Does it return to original shape? How much force is needed?” | Evidence generation through exploration: Learners gather firsthand observations to explain phenomenon. | Observe engagement, questioning, and observation quality. Note safety compliance. |
| Period 1, Min 35-40: Record observations in table: Material, What I did, What happened, Did it return to original shape? | Observation table template | Provide structured template. Prompt: “Be specific. Use measurements when possible.” | Documentation of evidence: Organized data supports later analysis and explanation. | Review observation tables for detail and accuracy. |
| Period 2, Min 0-10: Individual initial model: Draw and write to explain why different materials behave differently when forces are applied. | Initial model template (paper/digital) | Explain: “A model is your current best explanation using words and pictures. It’s okay if you’re not sure—we’ll revise it!” | Externalizing thinking: Initial models reveal current understanding and misconceptions to address. | Collect models. Analyze for understanding of forces, material structure, and causation. |
| Period 2, Min 10-15: Pair-Share models: Explain your model to partner. Discuss: What’s similar? Different? What questions do you have? | Partner discussion protocol | Circulate, listen to explanations. Prompt: “Can you explain why you drew it that way? What evidence supports your model?” | Peer feedback and revision: Learners refine thinking through explanation and questioning. | Listen for clarity of explanation and use of evidence. |
| Period 2, Min 15-25: Direct instruction: Teacher introduces material properties vocabulary (elasticity, plasticity, ductility, malleability, brittleness, hardness, stiffness, strength, toughness) with definitions, examples, and demonstrations. | Vocabulary cards, real examples, PPT/visual aids | Use visuals and real examples. Demonstrate each property. Ask: “Which materials from our exploration show this property?” | Vocabulary building with concrete referents: New terms connected to familiar observations. | Check understanding through questioning: “Give me an example of an elastic material.” |
| Period 2, Min 25-35: Classification activity: In groups, sort material samples by properties. Create a table showing which properties each material has. Test materials to confirm. | Material samples, classification table template | Provide clear criteria. Circulate, ask: “How do you know this material is ductile? What test can you do?” | Application of new vocabulary: Learners actively use terms to categorize evidence. | Review classification tables for accuracy. Observe group discussions and reasoning. |
| Period 2, Min 35-38: Start Driving Question Board: As a class, fill in “What do we know?” and “What do we need to know?” columns based on today’s learning. | DQB poster/digital board | Facilitate discussion. Record learner ideas. Prompt: “What did we learn today? What are you still wondering?” | Metacognitive tracking: Learners monitor their learning journey and identify gaps. | Note questions raised—these guide upcoming lessons. |
| Period 2, Min 38-40: Exit ticket: Define two material properties and give one example of each from real life. | Exit ticket slips/digital form | Distribute exit tickets. Collect as learners leave. | Individual accountability: Each learner demonstrates personal understanding. | Review exit tickets to identify who needs support and plan next lesson. |

### LESSON 2: Investigating Hooke’s Law (2 periods)

| Learner Experience | Resource Link | Teacher Moves | Sensemaking Strategy | Formative Assessment |
| --- | --- | --- | --- | --- |
| Period 1, Min 0-5: Review Lesson 1: Turn and talk—What is elasticity? What happens when you stretch a spring? Share with class. | Review prompts on board | Facilitate quick review. Ask: “Who can remind us what elasticity means?” Connect to today’s focus. | Connecting to prior learning: Activate relevant prior knowledge for new investigation. | Listen for accurate recall of elasticity concept. |
| Period 1, Min 5-10: PREDICT (POE): Individually predict: What will happen to spring extension as we add more weight? Draw a prediction graph (extension vs. mass). Write reasoning. | POE worksheet with graph template | Explain POE process. Emphasize: “There’s no wrong prediction—just explain your thinking.” | Prediction to engage prior knowledge: Learners commit to an idea, creating investment in outcome. | Review predictions to identify initial understanding of proportionality. |
| Period 1, Min 10-15: OBSERVE (POE): Watch teacher demonstration: Hang spring, measure original length, add masses one at a time (100g, 200g, 300g, 400g, 500g), measure total length each time. Record class data on board. | Spring, masses, retort stand, ruler, demonstration setup | Perform demonstration clearly. Think aloud: “Original length is 10 cm. With 100g, it’s now 12 cm. Extension is 2 cm.” Record data visibly. | Shared observation of evidence: All learners see same phenomenon to discuss. | Observe attention and note-taking. Check that learners record data accurately. |
| Period 1, Min 15-20: EXPLAIN (POE): Think-Pair-Share: Does the data match your prediction? Why or why not? What pattern do you see? | POE worksheet, class data | Facilitate discussion. Ask: “What surprises you? What pattern do you notice in the data?” | Confronting predictions with evidence: Cognitive dissonance drives learning. | Listen for recognition of linear relationship and reasoning quality. |
| Period 1, Min 20-35: Group investigation: In groups of 3-4, set up spring apparatus, measure original length, add masses incrementally (50g, 100g, 150g, 200g, 250g), measure and record extension for each mass. | Springs (various stiffnesses), masses, retort stands, rulers, data tables | Distribute materials. Model setup and measurement technique. Circulate, ask: “How do you calculate extension? Are you measuring carefully?” | Hands-on data collection: Learners generate their own evidence through investigation. | Observe setup, measurement technique, teamwork, and data recording. Check data tables. |
| Period 1, Min 35-40: Group discussion: Look at your data. What pattern do you see? Is extension proportional to mass (force)? | Data tables, group discussion prompts | Prompt groups: “Compare your first and last data points. Is the relationship linear?” | Pattern recognition from data: Learners analyze their evidence for relationships. | Listen to group discussions. Note whether groups identify proportionality. |
| Period 2, Min 0-10: Plot graph: On graph paper or using digital tool, plot Force (F = mg, y-axis) vs. Extension (x-axis). Use g = 10 m/s² for simplicity. | Graph paper, rulers, or digital graphing tool (Desmos, Excel) | Model graph setup: label axes, choose scale, plot points. Circulate to support graphing. | Visual representation of data: Graphs reveal patterns more clearly than tables. | Check graph construction: axes, labels, units, scale, plotted points. |
| Period 2, Min 10-15: Analyze graph: Think-Pair-Share: Is your graph linear? What does the slope represent? Draw best-fit line. | Completed graphs, rulers for best-fit line | Ask: “What shape is your graph? What does a straight line tell us about the relationship?” Introduce slope as spring constant. | Interpreting graphical relationships: Slope has physical meaning (stiffness). | Observe best-fit lines and listen for understanding that slope = spring constant. |
| Period 2, Min 15-25: Direct instruction: Introduce Hooke’s Law (F = kx). Explain: F = force applied, k = spring constant (stiffness), x = extension. Units: F in N, k in N/m, x in m. | Hooke’s Law formula on board, worked example | Present formula. Work through example: “If F = 2 N and x = 0.04 m, then k = F/x = 2/0.04 = 50 N/m.” | Formalizing the pattern: Mathematical relationship describes observed pattern. | Check understanding: “What does a large k value tell us about the spring?” |
| Period 2, Min 25-30: Calculate spring constant: From your graph slope or using k = F/x for each data point, calculate your spring’s spring constant. Average if using multiple points. | Calculators, data tables, graphs | Circulate, support calculations. Remind: “k = slope of your graph, or k = F/x.” | Applying mathematics to real data: Learners quantify their spring’s stiffness. | Check calculations for accuracy. Compare k values across groups. |
| Period 2, Min 30-35: Discuss elastic limit: Show or describe what happens if spring is stretched too far (permanent deformation). Define elastic limit. | Over-stretched spring example or video | Demonstrate or show video. Ask: “What happens if we exceed the elastic limit? Does Hooke’s Law still apply?” | Extending understanding to limitations: Hooke’s Law applies only within elastic limit. | Listen for understanding that excessive force causes permanent deformation. |
| Period 2, Min 35-38: Revise initial model: Add Hooke’s Law (F = kx) and elastic limit to your model from Lesson 1. Explain how this new knowledge improves your explanation. | Initial models (returned), revision materials | Return initial models. Prompt: “What can you add now? How does Hooke’s Law help explain spring behavior?” | Model revision based on evidence: Learners update thinking with new understanding. | Collect revised models. Check for inclusion of F = kx and elastic limit. |
| Period 2, Min 38-40: Exit ticket: Explain Hooke’s Law in your own words. Give one real-world example where Hooke’s Law applies. | Exit ticket slips/digital form | Distribute exit tickets. Collect as learners leave. | Individual accountability: Each learner articulates understanding. | Review exit tickets for conceptual understanding and ability to apply to real contexts. |

### LESSON 3: Understanding Stress and Strain (2 periods)

| Learner Experience | Resource Link | Teacher Moves | Sensemaking Strategy | Formative Assessment |
| --- | --- | --- | --- | --- |
| Period 1, Min 0-5: Review Hooke’s Law: Quick quiz—If F = 5 N and x = 0.1 m, what is k? Discuss: How can we compare different materials fairly? | Review question on board | Pose question, take responses. Transition: “F and x depend on size. We need better measures.” | Creating need for new concepts: Limitation of F and x motivates stress and strain. | Check quiz responses for Hooke’s Law mastery. |
| Period 1, Min 5-10: Demonstration: Show two wires (one thick, one thin, same material) with same force applied. Observe different extensions. Discuss: Why? | Two wires (different diameters), weights, retort stands | Perform demonstration. Ask: “Same force, different extensions. Why? What’s different about the wires?” | Cognitive dissonance: Same force, different results—size matters. | Listen for recognition that cross-sectional area affects extension. |
| Period 1, Min 10-20: Direct instruction: Introduce stress (σ = F/A) and strain (ε = ΔL/L₀). Explain: Stress = force per unit area (Pa or N/m²), strain = fractional change in length (no units). Show formulas, units, and meaning. | Formulas on board, visual diagrams, worked examples | Present concepts clearly. Use analogies: “Stress is like pressure—force spread over area. Strain is like percentage stretch.” Work through examples. | Introducing normalized measures: Stress and strain allow fair comparison across different sizes. | Check understanding: “What are the units of stress? Of strain?” |
| Period 1, Min 20-25: Think-Pair-Share: Why is stress more useful than force for comparing materials? Why is strain more useful than extension? | Discussion prompts on board | Facilitate discussion. Prompt: “Imagine comparing a thin wire and thick cable. Which is fairer—force or stress?” | Reasoning about utility of concepts: Learners justify why new measures are better. | Listen for understanding that stress and strain account for size differences. |
| Period 1, Min 25-35: Worked examples: Teacher models calculating stress and strain from given data (force, area, original length, extension). Learners follow along. | Worked example worksheet, document camera/board | Work through 2-3 examples step-by-step. Think aloud: “First, find area using A = πr². Then, σ = F/A.” | Modeling problem-solving process: Learners see expert thinking and calculation steps. | Observe learner engagement and note-taking. Ask: “What’s the next step?” |
| Period 1, Min 35-40: Practice problems: Individually solve 3 problems calculating stress and strain from given data. | Practice problem worksheet, calculators | Distribute problems. Circulate, provide hints. Remind: “Check your units!” | Independent practice: Learners apply formulas to build fluency. | Review solutions. Identify common errors (unit conversion, formula application). |
| Period 2, Min 0-5: Review practice problems: Go over solutions, address common errors (unit conversion, using diameter vs. radius, formula mix-ups). | Solutions on board, error examples | Highlight common mistakes. Ask: “Why is this wrong? How do we fix it?” | Error analysis: Learners learn from mistakes and clarify understanding. | Note who still struggles with calculations for follow-up support. |
| Period 2, Min 5-20: Group investigation: Measure wire sample (material assigned to group: copper, steel, or aluminum). Measure diameter with vernier calipers, calculate area. Measure original length. Hang wire, add masses incrementally, measure extension for each mass. | Wire samples, vernier calipers, masses, retort stands, meter sticks, data tables | Demonstrate vernier caliper use. Model measurement technique. Circulate, ensure accurate measurements. Ask: “How do you calculate area from diameter?” | Collecting real data for analysis: Learners generate authentic data to calculate stress and strain. | Observe measurement technique, data recording, and teamwork. Check data tables for completeness. |
| Period 2, Min 20-30: Calculate stress and strain: For each mass, calculate force (F = mg), stress (σ = F/A), and strain (ε = ΔL/L₀). Record in table. | Calculators, data tables, formula cards | Support calculations. Remind: “Use consistent units—convert mm to m.” | Applying formulas to real data: Learners quantify material behavior. | Check calculations for accuracy. Compare values across groups. |
| Period 2, Min 30-35: Compare results: Groups share their stress and strain values. Discuss: Which material has higher stress for the same strain? What does this tell us about the material? | Class data table on board, group presentations | Facilitate comparison. Ask: “Which material is stiffer—needs more stress for same strain?” | Comparative analysis: Patterns across materials reveal differences in stiffness. | Listen for reasoning about material stiffness based on stress-strain ratios. |
| Period 2, Min 35-38: Revise model: Add stress (σ = F/A) and strain (ε = ΔL/L₀) concepts to your model. Explain how these help compare materials. | Models (returned), revision materials | Return models. Prompt: “How do stress and strain improve our ability to compare materials?” | Model revision with new concepts: Learners integrate stress and strain into explanations. | Collect revised models. Check for accurate inclusion of formulas and explanations. |
| Period 2, Min 38-40: Exit ticket: Given force, area, original length, and extension, calculate stress and strain. | Exit ticket with calculation problem | Distribute exit ticket. Collect as learners leave. | Individual accountability: Each learner demonstrates calculation mastery. | Review exit tickets to identify who needs additional support. |

### LESSON 4: Exploring Young’s Modulus (2 periods)

| Learner Experience | Resource Link | Teacher Moves | Sensemaking Strategy | Formative Assessment |
| --- | --- | --- | --- | --- |
| Period 1, Min 0-5: Review stress and strain: Turn and talk—What do stress and strain tell us about materials? How are they different from force and extension? | Review prompts on board | Facilitate quick review. Ask: “Why are stress and strain more useful than force and extension?” | Activating prior knowledge: Connect to previous lesson before introducing new concept. | Listen for accurate understanding of stress and strain. |
| Period 1, Min 5-10: Pose question: How can we measure how “stiff” a material is? What would a “stiffness number” tell us? | Question on board, think time | Give think time. Take responses. Transition: “We need a measure that combines stress and strain.” | Creating cognitive need: Learners see need for a measure of stiffness. | Note initial ideas about stiffness. |
| Period 1, Min 10-20: Direct instruction: Introduce Young’s modulus (E = σ/ε). Explain: E = stress/strain, units = Pa (same as stress, since strain is unitless), meaning = stiffness (resistance to deformation). High E = stiff, low E = flexible. Show formula, units, examples. | Formula on board, material property table, visual aids | Present concept clearly. Use examples: “Steel has high E (~200 GPa)—very stiff. Rubber has low E (~0.01 GPa)—very flexible.” | Introducing stiffness measure: Young’s modulus quantifies material stiffness. | Check understanding: “What does a high Young’s modulus tell us?” |
| Period 1, Min 20-25: Worked examples: Teacher models calculating Young’s modulus from stress-strain data. Learners follow along. | Worked example worksheet, document camera/board | Work through 2 examples step-by-step. Think aloud: “We have σ = 50 MPa and ε = 0.001. So E = 50/0.001 = 50,000 MPa = 50 GPa.” | Modeling calculation process: Learners see how to calculate E from data. | Observe engagement and note-taking. Ask: “What’s the next step?” |
| Period 1, Min 25-35: Practice problems: Individually solve 3 problems calculating Young’s modulus from given stress and strain values. | Practice problem worksheet, calculators | Distribute problems. Circulate, provide support. Remind: “Watch your units—convert MPa to Pa if needed.” | Independent practice: Learners build fluency with E calculations. | Review solutions. Identify common errors (unit conversion, division errors). |
| Period 1, Min 35-40: Think-Pair-Share: What does a high E value tell us about a material? A low E value? Give examples of materials with high and low E. | Discussion prompts on board | Facilitate discussion. Ask: “Would you want high or low E for a bridge cable? For a rubber band?” | Reasoning about meaning: Learners connect E values to material behavior and applications. | Listen for understanding that high E = stiff, low E = flexible. |
| Period 2, Min 0-10: Calculate Young’s modulus from Lesson 3 data: Use your stress and strain values from wire investigation. Calculate E for each data point, then average. | Lesson 3 data tables, calculators, formula cards | Remind: “E = σ/ε for each data point. Then find the average E.” Circulate to support. | Applying formula to real data: Learners quantify their material’s stiffness. | Check calculations for accuracy. Compare E values across groups. |
| Period 2, Min 10-15: Compare with reference table: Look up Young’s modulus for your material (copper, steel, aluminum) in reference table. How close is your calculated value? Discuss sources of error. | Reference table of Young’s modulus values | Provide reference table. Ask: “How does your value compare? What might cause differences?” | Validating results: Learners compare experimental results with accepted values. | Note accuracy of experimental values and understanding of error sources. |
| Period 2, Min 15-25: Group activity—Material selection: Given scenarios (bridge cable, shock absorber, building beam, phone case), groups select best material based on Young’s modulus values from reference table. Justify choices. | Scenario cards, reference table, justification worksheet | Distribute scenarios. Prompt: “Consider what the material needs to do. Should it be stiff or flexible?” | Application to real-world contexts: Learners use E values to make informed material choices. | Review justifications for reasoning quality and use of E values. |
| Period 2, Min 25-30: Groups present: Each group shares their scenario, material choice, and justification. Class discusses and questions. | Presentation space, scenario cards | Facilitate presentations. Encourage questions: “Why did you choose that material? What would happen if you used a different one?” | Peer learning and critique: Learners evaluate reasoning and learn from others’ applications. | Assess presentation clarity and depth of justification. |
| Period 2, Min 30-35: Discuss stiffness vs. strength: Can a material be stiff but weak (brittle)? Strong but flexible (tough)? Give examples (glass = stiff but brittle, rubber = flexible but not strong, steel = both stiff and strong). | Examples and diagrams on board | Clarify distinctions. Use examples: “Glass has high E (stiff) but breaks easily (brittle). Steel has high E and high strength.” | Clarifying related concepts: Learners distinguish between stiffness (E) and strength (breaking force). | Listen for understanding that stiffness and strength are different properties. |
| Period 2, Min 35-38: Update DQB: What have we learned so far? What questions do we still have? Add to “What we know” and “What we need to know” columns. | DQB poster/digital board | Facilitate discussion. Record learner contributions. Prompt: “We know about properties, Hooke’s Law, stress, strain, and Young’s modulus. What’s left to understand?” | Metacognitive tracking: Learners monitor progress toward answering driving question. | Note remaining questions to address in upcoming lessons. |
| Period 2, Min 38-40: Exit ticket: Explain what Young’s modulus is and why it’s useful for comparing materials. Give one example. | Exit ticket slips/digital form | Distribute exit tickets. Collect as learners leave. | Individual accountability: Each learner articulates understanding of E. | Review exit tickets for conceptual understanding and application ability. |

### LESSON 5: Analyzing Stress-Strain Graphs (2 periods)

| Learner Experience | Resource Link | Teacher Moves | Sensemaking Strategy | Formative Assessment |
| --- | --- | --- | --- | --- |
| Period 1, Min 0-5: Review: Quick quiz—What is stress? Strain? Young’s modulus? How are they related? | Review questions on board | Pose questions, take responses. Transition: “Today we’ll see how stress and strain relate graphically.” | Activating prior knowledge: Ensure understanding of concepts before graphical analysis. | Check quiz responses for mastery of stress, strain, and E. |
| Period 1, Min 5-10: Display stress-strain graph for steel: Give 2 minutes to observe silently. Then Think-Pair-Share: What do you notice? What questions do you have? | Stress-strain graph (large display), observation prompts | Display graph prominently. Give think time. Facilitate sharing: “What patterns do you see?” | Observation before instruction: Learners generate questions and notice features before being told. | Listen for observations about shape, regions, and features. |
| Period 1, Min 10-25: Direct instruction: Teach key features of stress-strain graphs: 1) Elastic region (linear, material returns to original shape), 2) Elastic limit/yield point (end of elastic region), 3) Plastic region (permanent deformation), 4) Ultimate tensile strength (maximum stress), 5) Breaking point (fracture), 6) Area under curve (energy absorbed = toughness). Annotate graph as you explain. | Annotated stress-strain graph, colored markers, visual aids | Point to each feature on graph. Use colors to highlight regions. Explain: “In elastic region, remove stress and material returns. Past elastic limit, deformation is permanent.” | Building graph interpretation skills: Learners learn to read and extract information from stress-strain graphs. | Check understanding: “Point to the elastic limit. What happens after this point?” |
| Period 1, Min 25-35: Guided practice: Distribute blank stress-strain graph. Learners annotate it with all key features (elastic region, elastic limit, plastic region, ultimate tensile strength, breaking point, label axes). Use colored pencils. | Blank stress-strain graph, colored pencils, annotation checklist | Circulate, check annotations. Prompt: “Label each region. What does the slope in the elastic region represent?” (Answer: Young’s modulus) | Active annotation: Learners internalize graph features through labeling. | Check annotated graphs for accuracy and completeness. |
| Period 1, Min 35-40: Think-Pair-Share: What’s the difference between elastic and plastic deformation? Give real-world examples of each. | Discussion prompts on board | Facilitate discussion. Ask: “When you stretch a rubber band and release, is that elastic or plastic? What about bending a paperclip?” | Connecting concepts to experience: Learners relate graph features to familiar phenomena. | Listen for accurate distinction between elastic (reversible) and plastic (permanent) deformation. |
| Period 2, Min 0-10: Compare stress-strain graphs: Display graphs for steel, rubber, glass, and aluminum side-by-side. In groups, analyze: Which material is stiffest (steepest slope)? Strongest (highest ultimate tensile strength)? Toughest (largest area under curve)? Most brittle (breaks with little plastic deformation)? | Multiple stress-strain graphs, comparison worksheet | Display graphs clearly. Prompt: “Look at slopes, peak heights, and areas. What do these tell us?” | Comparative analysis: Learners extract and compare information across materials. | Review group analyses for accuracy in identifying stiffness, strength, toughness, and brittleness. |
| Period 2, Min 10-15: Group discussion: Share findings. Discuss: Steel = stiff and strong, rubber = flexible and tough, glass = stiff but brittle, aluminum = moderate stiffness and strength. | Group findings, class discussion | Facilitate sharing. Ask: “Why is glass brittle even though it’s stiff? What makes rubber tough?” | Synthesizing graph information: Learners connect graph features to material properties. | Listen for reasoning connecting graph features to material behavior. |
| Period 2, Min 15-25: Application scenarios: Match materials to uses based on stress-strain graph features. Scenarios: 1) Construction beam (needs high strength and stiffness = steel), 2) Shock absorber (needs large elastic region = rubber/spring), 3) Safety glass (needs high toughness = treated glass), 4) Wire drawing (needs ductility = copper). Groups justify choices. | Scenario cards, stress-strain graphs, justification worksheet | Distribute scenarios. Prompt: “What property is most important for this use? Which graph shows that property?” | Application to authentic contexts: Learners use graph analysis to make informed material selections. | Review justifications for reasoning quality and correct use of graph features. |
| Period 2, Min 25-30: Discuss energy storage: Explain that area under stress-strain curve represents energy absorbed before breaking (toughness). Why does this matter? (Shock absorbers, safety equipment, crash protection) | Stress-strain graphs with shaded areas, real-world examples | Shade area under curve. Explain: “More area = more energy absorbed = tougher material. Important for safety!” | Connecting graph feature to function: Learners understand practical significance of area under curve. | Check understanding: “Why do we want high toughness in a helmet?” |
| Period 2, Min 30-35: Revise model: Create comprehensive explanation of material behavior. Include: material properties, Hooke’s Law, stress, strain, Young’s modulus, stress-strain graph features (elastic/plastic regions, elastic limit, breaking point). Use diagrams and labels. | Comprehensive model template, previous models, colored pencils | Return all previous models. Prompt: “Combine everything we’ve learned into one complete explanation. How do materials respond to forces?” | Synthesis of all concepts: Learners integrate entire unit’s learning into coherent explanation. | Collect comprehensive models. Assess for inclusion of all concepts and coherence. |
| Period 2, Min 35-38: Update DQB: Can we answer the driving question now? As a class, draft answer: “Materials change shape when forces are applied. Elastic materials return to original shape (within elastic limit) because of molecular structure. Plastic materials deform permanently. Stress, strain, and Young’s modulus help us measure and compare material behavior.” | DQB poster/digital board | Facilitate discussion. Record class answer. Celebrate: “We can now answer our driving question!” | Answering driving question: Learners see how investigation has led to comprehensive understanding. | Assess class answer for completeness and accuracy. |
| Period 2, Min 38-40: Exit ticket: Sketch a stress-strain graph and label: elastic region, elastic limit, plastic region, ultimate tensile strength, breaking point. | Exit ticket with blank axes | Distribute exit tickets. Collect as learners leave. | Individual accountability: Each learner demonstrates graph interpretation mastery. | Review exit ticket graphs for accuracy and completeness of labels. |

### LESSON 6: Applications, Synthesis, and Assessment (2 periods)

| Learner Experience | Resource Link | Teacher Moves | Sensemaking Strategy | Formative Assessment |
| --- | --- | --- | --- | --- |
| Period 1, Min 0-5: Review unit: Quick gallery walk—look at DQB, previous models, and key concepts posted around room. Reflect: What have we learned about materials and forces? | DQB, posted models and concepts, gallery walk protocol | Set up gallery walk. Prompt: “Walk around, review our learning journey. What stands out?” | Reviewing learning journey: Learners reflect on progression from initial ideas to current understanding. | Observe engagement and reflections shared. |
| Period 1, Min 5-15: Case study exploration: Groups receive Kenya-specific application scenarios. Group 1: Building construction—steel reinforcement bars in concrete (why steel? what properties matter?). Group 2: Matatu shock absorbers and suspension (why springs/rubber? what properties matter?). Group 3: Jua Kali metalworking—selecting materials for tools and products (ductility, malleability, strength). Group 4: Bridge construction—cable materials (high strength, high Young’s modulus, ductility for safety). Groups read case study, discuss, and analyze. | Case study cards with images and descriptions, analysis worksheet | Distribute case studies. Prompt: “Read carefully. Identify what material properties are most important and why.” Circulate to support. | Application to authentic Kenya contexts: Learners connect learning to real-world local applications. | Observe group discussions for depth of analysis and use of concepts. |
| Period 1, Min 15-25: Group analysis: Groups answer: 1) What material properties are most important for this application? 2) Why are these properties critical? 3) What would happen if wrong materials were used? (safety, cost, durability implications). Use evidence from unit (formulas, graphs, properties). | Analysis worksheet, reference materials (property tables, graphs) | Circulate, ask probing questions: “Why is high Young’s modulus important for bridge cables? What if we used rubber instead?” | Critical thinking about material selection: Learners apply knowledge to evaluate real-world choices. | Review group analyses for reasoning quality, use of evidence, and consideration of consequences. |
| Period 1, Min 25-30: Group presentations: Each group presents their case study, material property analysis, and justification. Class asks questions and discusses. | Presentation space, case study materials | Facilitate presentations. Encourage questions: “What evidence supports your choice? What trade-offs exist?” | Peer learning and critique: Learners learn from diverse applications and evaluate reasoning. | Assess presentation clarity, depth of analysis, and response to questions. |
| Period 1, Min 30-38: Individual final model: Create comprehensive diagram and written explanation answering the driving question: “How do materials change shape when forces are applied, and why do some return to their original shape while others don’t?” Include: material properties (elasticity, plasticity, etc.), Hooke’s Law (F = kx), stress (σ = F/A), strain (ε = ΔL/L₀), Young’s modulus (E = σ/ε), stress-strain graph with labeled features, real-world examples. | Final model template (large paper or digital), colored pencils/markers, reference materials | Explain expectations: “This is your complete explanation using everything we’ve learned. Make it clear, accurate, and detailed.” Circulate to support. | Comprehensive synthesis: Learners integrate all unit concepts into coherent, evidence-based explanation. | Collect final models for summative assessment using rubric (see Lesson 6 plan). |
| Period 1, Min 38-40: Complete DQB: As a class, finalize answer to driving question. Celebrate learning journey. | DQB poster/digital board, markers | Facilitate final DQB completion. Read final answer aloud. Celebrate: “Look how far we’ve come from our initial ideas!” | Metacognitive closure: Learners see growth from initial to final understanding. | Note participation and contributions to final answer. |
| Period 2, Min 0-30: Written assessment: Complete individually. Sections: 1) Multiple choice (10 questions on material properties, Hooke’s Law, stress, strain, Young’s modulus, graph interpretation), 2) Calculations (4 problems: calculate spring constant, stress, strain, Young’s modulus), 3) Graph interpretation (2 questions: identify features, compare materials), 4) Short answer (2 questions: explain elastic vs. plastic deformation, describe material selection for given application). | Written assessment packet, calculators, formula sheet (if provided) | Distribute assessments. Clarify instructions. Monitor for academic integrity. Provide time warnings. | Individual demonstration of mastery: Learners show knowledge, skills, and application ability. | Grade assessments using answer key and rubric. Identify strengths and areas for improvement. |
| Period 2, Min 30-38: Practical task: Given 3 material samples (e.g., steel wire, rubber band, plastic strip) and a scenario (e.g., “You need to design a safety cable for a construction site”), select the best material and justify your choice based on mechanical properties. Consider: strength, elasticity, ductility, Young’s modulus, cost, availability in Kenya. | Material samples, scenario card, justification worksheet | Distribute materials and scenario. Prompt: “Test the materials if needed. Justify your choice using specific properties and evidence.” | Authentic performance task: Learners apply knowledge to make and justify real-world material selection. | Assess using rubric: appropriateness of selection, quality of justification, use of mechanical properties, consideration of context. |
| Period 2, Min 38-40: Reflection: Write individual reflection: 1) What was most interesting in this unit? 2) What was most challenging? 3) How will you use this knowledge in the future? 4) What questions do you still have? | Reflection prompts (paper or digital) | Provide reflection prompts. Encourage honesty. Collect reflections. | Metacognitive reflection: Learners consider their learning experience and future applications. | Review reflections for insights into learner experience, remaining questions, and unit effectiveness. |

## SECTION D: DIFFERENTIATION STRATEGIES

### For Struggling Learners:

Conceptual Support: - Provide vocabulary cards with definitions, images, and examples in English and Kiswahili - Use concrete manipulatives and hands-on materials extensively - Break complex tasks into smaller, manageable steps with checklists - Offer graphic organizers for note-taking and concept mapping - Provide sentence starters for discussions and written explanations - Use color-coding to distinguish different concepts (e.g., stress = blue, strain = green)

Skill Support: - Provide formula cards with worked examples for all calculations - Offer step-by-step calculation guides with spaces to fill in values - Use calculators and allow extra time for mathematical tasks - Provide partially completed data tables and graphs - Offer one-on-one or small group support during investigations - Pre-teach key vocabulary and concepts before lessons

Assessment Modifications: - Allow oral explanations instead of written for some tasks - Provide extended time for assessments - Offer simplified scenarios with fewer variables - Allow use of formula sheets and reference materials - Break assessments into smaller chunks over multiple days - Provide multiple-choice options for some open-ended questions

Social-Emotional Support: - Pair with supportive peers for Think-Pair-Share - Celebrate small successes and progress - Provide frequent, specific, positive feedback - Offer alternative ways to demonstrate understanding (drawing, modeling, oral presentation) - Create a safe environment for asking questions and making mistakes

### For Advanced Learners:

Conceptual Extensions: - Explore advanced material properties: viscoelasticity, creep, fatigue, fracture mechanics - Investigate composite materials and their stress-strain behavior - Research how temperature affects mechanical properties - Study atomic/molecular basis of elasticity and plasticity - Explore anisotropic materials (properties vary with direction) - Investigate smart materials (shape-memory alloys, piezoelectric materials)

Skill Extensions: - Calculate energy stored in springs using E = ½kx² - Determine area under stress-strain curves to calculate toughness - Use calculus to derive relationships between stress, strain, and Young’s modulus - Design and conduct original investigations (e.g., effect of temperature on spring constant) - Use advanced graphing and data analysis software - Create computer simulations of material behavior

Application Extensions: - Design a structure (bridge, building, vehicle component) with justified material selections - Research and present on cutting-edge materials (graphene, carbon nanotubes, metamaterials) - Investigate material failures in real-world disasters (bridge collapses, building failures) and propose improvements - Explore biomimicry: how nature’s materials (spider silk, bone, wood) inspire engineering - Conduct cost-benefit analysis of material choices for Kenya-specific projects - Create educational materials (videos, infographics) explaining mechanical properties to younger students

Independent Projects: - Research project on a specific material or application - Design challenge: create a device that uses specific mechanical properties - Community project: assess materials used in local construction and make recommendations - Mentorship: support struggling peers in understanding concepts - Literature review: read and summarize scientific papers on materials science

### For English Language Learners (ELLs):

Language Support: - Provide bilingual vocabulary lists (English-Kiswahili) with visual representations - Use visual aids extensively: diagrams, photos, videos, demonstrations - Offer sentence frames for discussions and writing: “I predict that _____ because _____.” “The data shows that _____.” - Allow use of translation tools and bilingual dictionaries - Provide written instructions in addition to oral instructions - Use gestures, demonstrations, and physical models to support verbal explanations

Comprehension Support: - Pre-teach key vocabulary before each lesson - Provide graphic organizers and concept maps - Use think-alouds to model thinking processes - Offer audio recordings of instructions and key content - Provide extra processing time for questions and tasks - Use peer support: pair with bilingual peers when possible

Expression Support: - Allow responses in home language with translation support - Accept drawings, diagrams, and models as valid forms of explanation - Provide multiple-choice or fill-in-the-blank options alongside open-ended questions - Offer oral explanations instead of written when appropriate - Use collaborative structures (Think-Pair-Share) to practice language in low-stakes settings - Celebrate multilingualism and encourage use of home language as a resource

Assessment Modifications: - Provide assessments in simplified language - Allow use of bilingual dictionaries during assessments - Offer extended time for reading and writing tasks - Provide visual supports on assessments (diagrams, graphs, images) - Allow oral responses for some assessment items - Focus assessment on content understanding rather than language proficiency

### Differentiation by Learning Style:

Visual Learners: - Provide diagrams, graphs, charts, and color-coded notes - Use videos and animations to illustrate concepts - Encourage creation of visual models and concept maps - Highlight key information with colors and symbols

Auditory Learners: - Use verbal explanations and discussions extensively - Provide audio recordings of key content - Encourage Think-Pair-Share and group discussions - Use mnemonic devices and verbal patterns

Kinesthetic Learners: - Maximize hands-on investigations and manipulatives - Encourage movement during learning (gallery walks, station rotations) - Use physical models and demonstrations - Allow building and creating as forms of explanation

Reading/Writing Learners: - Provide written materials, articles, and textbook readings - Encourage note-taking and written reflections - Offer opportunities for written explanations and reports - Provide reading materials at various levels

## SECTION E: FINAL SYNTHESIS AND ASSESSMENT

### 1. Summative Assessment Overview

The summative assessment for this unit consists of four components designed to evaluate learners’ knowledge, skills, and ability to apply understanding of mechanical properties of materials:

Component 1: Final Explanatory Model (30 points) Component 2: Written Assessment (40 points) Component 3: Practical Task (20 points) Component 4: Participation and Process (10 points)

Total: 100 points

### 2. Component 1: Final Explanatory Model (30 points)

Task: Create a comprehensive diagram and written explanation answering the driving question: “How do materials change shape when forces are applied, and why do some return to their original shape while others don’t?”

Requirements: - Visual representation (diagram, concept map, or annotated illustration) - Written explanation (1-2 pages) - Must include: - Material properties (elasticity, plasticity, ductility, malleability, brittleness, hardness, stiffness, strength, toughness) - Hooke’s Law (F = kx) with explanation - Stress (σ = F/A) and strain (ε = ΔL/L₀) with explanations - Young’s modulus (E = σ/ε) with explanation - Stress-strain graph with labeled features (elastic region, elastic limit, plastic region, ultimate tensile strength, breaking point) - Real-world examples from Kenya contexts - Clear, accurate scientific terminology

Rubric:

| Criterion | Excellent (5 points) | Proficient (3-4 points) | Developing (1-2 points) | Beginning (0 points) |
| --- | --- | --- | --- | --- |
| Addresses Driving Question | Completely and clearly answers how and why materials change shape and return (or don’t return) to original shape | Mostly answers the question with minor gaps | Partially answers the question with significant gaps | Does not address the driving question |
| Inclusion of Key Concepts (×2) | Includes ALL concepts (properties, Hooke’s Law, stress, strain, Young’s modulus, stress-strain graphs) with accurate explanations | Includes MOST concepts (5-6) with mostly accurate explanations | Includes SOME concepts (3-4) with some inaccuracies | Includes FEW concepts (0-2) or major inaccuracies |
| Scientific Terminology | Uses all key terms accurately and appropriately throughout | Uses most key terms accurately with minor errors | Uses some key terms with several errors | Uses few key terms or major errors |
| Explanations and Examples | Provides clear, detailed explanations with relevant Kenya-specific examples | Provides adequate explanations with some examples | Provides limited explanations with few examples | Provides unclear or no explanations and examples |
| Visual Representation | Visual is clear, well-organized, accurately labeled, and enhances understanding | Visual is mostly clear and labeled, supports understanding | Visual is present but unclear or poorly labeled | Visual is absent or does not support understanding |

Total: 30 points

### 3. Component 2: Written Assessment (40 points)

Section A: Multiple Choice (10 points, 1 point each)

- Which property describes a material’s ability to return to its original shape after deformation?

    - Plasticity

    - Elasticity

    - Ductility

    - Brittleness

- Hooke’s Law states that:

    - Force is inversely proportional to extension

    - Force equals mass times acceleration

    - Force is directly proportional to extension

    - Force is independent of extension

- The spring constant (k) in Hooke’s Law is measured in:

    - Newtons (N)

    - Meters (m)

    - Newtons per meter (N/m)

    - Pascals (Pa)

- Stress is defined as:

    - Force per unit area

    - Change in length per original length

    - Force times extension

    - Mass per unit volume

- Strain is:

    - Measured in Pascals

    - A dimensionless quantity

    - Equal to stress divided by Young’s modulus

    - The same as stress

- Young’s modulus measures a material’s:

    - Strength

    - Hardness

    - Stiffness

    - Toughness

- On a stress-strain graph, the elastic limit is:

    - The point where the material breaks

    - The maximum stress the material can withstand

    - The point beyond which deformation is permanent

    - The slope of the graph

- A material that breaks without significant plastic deformation is:

    - Ductile

    - Malleable

    - Brittle

    - Elastic

- The area under a stress-strain curve represents:

    - Young’s modulus

    - Stiffness

    - Energy absorbed (toughness)

    - Elastic limit

- Which material would be best for a spring in a matatu shock absorber?

    - Glass (high stiffness, brittle)

    - Steel (high stiffness, elastic, strong)

    - Clay (plastic, weak)

    - Wood (moderate stiffness, anisotropic)

Answer Key: 1-b, 2-c, 3-c, 4-a, 5-b, 6-c, 7-c, 8-c, 9-c, 10-b

Section B: Calculations (20 points, 5 points each)

Problem 1: A spring has an original length of 15 cm. When a force of 4 N is applied, it extends to 19 cm. Calculate the spring constant (k).

Solution: - Extension (x) = 19 cm - 15 cm = 4 cm = 0.04 m - F = kx → k = F/x = 4 N / 0.04 m = 100 N/m

Problem 2: A wire with a cross-sectional area of 2 × 10⁻⁶ m² is subjected to a tensile force of 50 N. Calculate the stress in the wire.

Solution: - σ = F/A = 50 N / (2 × 10⁻⁶ m²) = 25 × 10⁶ Pa = 25 MPa

Problem 3: A metal rod has an original length of 2.0 m. When a force is applied, it extends by 2 mm. Calculate the strain.

Solution: - ΔL = 2 mm = 0.002 m - L₀ = 2.0 m - ε = ΔL/L₀ = 0.002 m / 2.0 m = 0.001 (or 0.1%)

Problem 4: A material experiences a stress of 150 MPa and a strain of 0.0015. Calculate Young’s modulus for this material.

Solution: - E = σ/ε = 150 MPa / 0.0015 = 100,000 MPa = 100 GPa

Grading Rubric for Calculations (per problem): - Correct formula identified: 1 point - Correct substitution of values: 1 point - Correct calculation: 2 points - Correct units: 1 point

Section C: Graph Interpretation (6 points, 3 points each)

Question 1: The graph below shows stress-strain curves for three materials (A, B, C).

[Graph would show: Material A = steep slope, high ultimate strength, small plastic region (steel); Material B = gentle slope, large elastic region, moderate strength (rubber); Material C = steep slope, breaks at elastic limit (glass)]

- Which material is the stiffest? Explain how you know. (1.5 points)

- Which material is the most brittle? Explain how you know. (1.5 points)

Answer: a) Material A (or C) is stiffest because it has the steepest slope in the elastic region, indicating highest Young’s modulus. b) Material C is most brittle because it breaks at or near the elastic limit with little to no plastic deformation.

Question 2: On the same graph: a) Label the elastic region, elastic limit, and plastic region for Material A. (1.5 points) b) Which material would be best for a shock absorber? Explain. (1.5 points)

Answer: a) [Correct labeling on graph] b) Material B would be best because it has a large elastic region, allowing it to absorb energy and return to original shape repeatedly.

Section D: Short Answer (4 points, 2 points each)

Question 1: Explain the difference between elastic deformation and plastic deformation. Give one example of each from everyday life.

Answer (2 points): - Elastic deformation is temporary—material returns to original shape when force is removed (e.g., stretching a rubber band). (1 point) - Plastic deformation is permanent—material does not return to original shape (e.g., bending a paperclip). (1 point)

Question 2: Why is it important to consider Young’s modulus when selecting materials for building construction in Kenya? Give a specific example.

Answer (2 points): - Young’s modulus indicates stiffness—how much a material will deform under load. (0.5 points) - In construction, materials must be stiff enough to support loads without excessive deformation (safety and structural integrity). (0.5 points) - Example: Steel reinforcement bars have high Young’s modulus, making them suitable for supporting concrete in buildings and bridges. (1 point)

### 4. Component 3: Practical Task (20 points)

Task: You are a materials engineer working on a construction project in Kenya. You need to select a material for safety cables that will support workers on a high-rise building. You are given three material samples: - Sample A: Steel wire - Sample B: Rubber cord - Sample C: Plastic rope

Instructions: 1. Examine and test the three material samples (stretch, bend, observe behavior). 2. Select the best material for safety cables. 3. Write a justification (1 paragraph) explaining your choice based on mechanical properties (strength, elasticity, ductility, Young’s modulus, toughness). 4. Consider practical factors: cost, availability in Kenya, durability, safety.

Rubric:

| Criterion | Excellent (5 points) | Proficient (3-4 points) | Developing (1-2 points) | Beginning (0 points) |
| --- | --- | --- | --- | --- |
| Material Selection Appropriateness (×2) | Selects most appropriate material (steel) with clear reasoning | Selects appropriate material with adequate reasoning | Selects less appropriate material or weak reasoning | Selects inappropriate material or no reasoning |
| Use of Mechanical Properties (×2) | Justification includes multiple relevant properties (strength, Young’s modulus, ductility, toughness) with accurate explanations | Justification includes some relevant properties with mostly accurate explanations | Justification includes few properties or inaccurate explanations | Justification lacks mechanical properties or major inaccuracies |
| Consideration of Kenya Context | Thoroughly considers cost, availability, durability, and safety in Kenya | Adequately considers some practical factors | Minimally considers practical factors | Does not consider practical factors |
| Communication Clarity | Justification is clear, well-organized, and persuasive | Justification is mostly clear and organized | Justification is unclear or poorly organized | Justification is absent or incomprehensible |

Total: 20 points

### 5. Component 4: Participation and Process (10 points)

This component assesses learners’ engagement and growth throughout the unit.

Rubric:

| Criterion | Excellent (3 points) | Proficient (2 points) | Developing (1 point) | Beginning (0 points) |
| --- | --- | --- | --- | --- |
| DQB Contributions | Consistently contributes thoughtful questions and ideas to DQB throughout unit | Regularly contributes to DQB with some thoughtful input | Occasionally contributes to DQB with limited input | Rarely or never contributes to DQB |
| Laboratory Investigation Participation | Actively participates in all investigations, collaborates effectively, follows safety procedures | Participates in most investigations, collaborates adequately, mostly follows safety | Participates minimally in investigations, limited collaboration, some safety issues | Does not participate meaningfully or violates safety procedures |
| Model Revisions Showing Growth (×2) | Models show clear progression from initial to final understanding, incorporating all new concepts | Models show some progression with most new concepts incorporated | Models show limited progression with few new concepts | Models show no progression or are incomplete |
| Think-Pair-Share Engagement (×2) | Consistently engages in all Think-Pair-Share activities, listens actively, shares thoughtfully | Regularly engages in Think-Pair-Share, adequate listening and sharing | Occasionally engages in Think-Pair-Share, limited listening or sharing | Rarely engages in Think-Pair-Share or disruptive |

Total: 10 points

### 6. Overall Assessment Summary

| Component | Points | Percentage |
| --- | --- | --- |
| Final Explanatory Model | 30 | 30% |
| Written Assessment | 40 | 40% |
| Practical Task | 20 | 20% |
| Participation and Process | 10 | 10% |
| TOTAL | 100 | 100% |

Grading Scale: - A: 90-100 points (Exceeds Expectations) - B: 80-89 points (Meets Expectations) - C: 70-79 points (Approaches Expectations) - D: 60-69 points (Below Expectations) - E: Below 60 points (Does Not Meet Expectations)

### 7. Post-Assessment Actions

For Teachers: 1. Grade all assessment components using provided rubrics within one week. 2. Analyze results to identify: - Concepts mastered by most learners - Concepts requiring reteaching or reinforcement - Individual learners needing intervention or extension 3. Provide feedback to learners: - Return assessments with specific, actionable comments - Highlight strengths and areas for growth - Offer opportunities for revision or reassessment where appropriate 4. Reflect on instruction: - What worked well in this unit? - What challenges did learners face? - How can the unit be improved for future implementation? 5. Plan next steps: - Remediation for struggling learners (small group instruction, additional practice) - Extension for advanced learners (independent projects, mentorship) - Connections to upcoming units

For Learners: 1. Review feedback on all assessment components. 2. Reflect on learning: - What did you learn well? - What do you still find challenging? - How can you improve? 3. Set goals for future learning in physics. 4. Apply knowledge to real-world observations and problems.

### 8. Connections to Future Learning

This unit on Mechanical Properties of Materials provides foundational knowledge for future physics topics:

Grade 10 (Later in the year): - Work, Energy, and Power: Energy stored in springs (elastic potential energy = ½kx²) - Pressure: Stress as a type of pressure, applications in hydraulics

Grade 11: - Circular Motion and Gravitation: Tension in cables and strings - Oscillations and Waves: Simple harmonic motion of springs and elastic materials

Grade 12: - Advanced Mechanics: Rotational motion, torque, and material properties in structures - Modern Physics: Atomic and molecular basis of material properties

Beyond School: - Engineering: Materials science, structural engineering, mechanical engineering - Architecture: Building design and material selection - Manufacturing: Jua Kali sector, industrial production - Research: Materials science, nanotechnology, biomaterials

## APPENDICES

### Appendix A: Key Formulas Summary

| Concept | Formula | Variables | Units |
| --- | --- | --- | --- |
| Hooke’s Law | F = kx | F = force, k = spring constant, x = extension | F: N, k: N/m, x: m |
| Stress | σ = F/A | σ = stress, F = force, A = cross-sectional area | σ: Pa or N/m², F: N, A: m² |
| Strain | ε = ΔL/L₀ | ε = strain, ΔL = change in length, L₀ = original length | ε: dimensionless (no units) |
| Young’s Modulus | E = σ/ε | E = Young’s modulus, σ = stress, ε = strain | E: Pa or N/m² |
| Force (Weight) | F = mg | F = force, m = mass, g = gravitational acceleration (≈10 m/s²) | F: N, m: kg, g: m/s² |
| Area (Circle) | A = πr² | A = area, r = radius | A: m², r: m |
| Elastic Potential Energy | E = ½kx² | E = energy, k = spring constant, x = extension | E: J, k: N/m, x: m |

### Appendix B: Material Properties Reference Table

| Material | Young’s Modulus (GPa) | Tensile Strength (MPa) | Density (kg/m³) | Key Properties |
| --- | --- | --- | --- | --- |
| Steel | 200 | 400-550 | 7850 | High strength, stiff, ductile, tough |
| Aluminum | 69 | 90-300 | 2700 | Moderate strength, lightweight, ductile, corrosion-resistant |
| Copper | 120 | 220 | 8960 | Ductile, malleable, good conductor |
| Rubber | 0.01-0.1 | 15-30 | 1100-1200 | Very flexible, elastic, tough |
| Glass | 70 | 50 | 2500 | Stiff, brittle, transparent |
| Concrete | 30 | 3-5 (compression: 20-40) | 2400 | Stiff, strong in compression, weak in tension, brittle |
| Wood (along grain) | 10-15 | 50-100 | 400-700 | Moderate stiffness, anisotropic, renewable |
| Plastic (PVC) | 2-4 | 40-60 | 1380 | Flexible, lightweight, durable |
| Bone | 15-20 | 130 | 1900 | Stiff, tough, anisotropic |
| Spider Silk | 10 | 1000 | 1300 | Very strong, elastic, tough |

### Appendix C: Safety Guidelines for Laboratory Investigations

General Safety Rules: 1. Wear safety goggles at all times during investigations. 2. Clear workspace of unnecessary items to prevent tripping or knocking over equipment. 3. Handle equipment carefully: Springs, wires, and masses can cause injury if mishandled. 4. Secure apparatus: Ensure retort stands are stable and clamps are tight. 5. Keep feet clear of hanging masses to prevent injury if they fall. 6. Report accidents immediately to the teacher, no matter how minor. 7. Follow instructions carefully and ask questions if unsure. 8. Work collaboratively and watch out for each other’s safety.

Specific Hazards and Precautions:

| Hazard | Precaution |
| --- | --- |
| Falling masses | Secure masses properly, keep feet clear, use appropriate mass sizes |
| Sharp wire ends | Handle wires carefully, file sharp ends if necessary, wear gloves if needed |
| Breaking glass | Teacher demonstration only, use safety screen, wear goggles, dispose of broken glass in designated container |
| Over-stretched springs | Do not exceed recommended mass limits, stop if spring shows permanent deformation |
| Tipping retort stands | Use heavy bases, clamp securely, do not overload |
| Pinching fingers | Be careful when attaching clamps and masses, keep fingers clear of moving parts |

Emergency Procedures: - Minor cuts: Wash with water, apply pressure, bandage, report to teacher - Serious injury: Stop all work, call for help immediately, do not move injured person unless necessary - Broken glass: Do not touch with bare hands, notify teacher, use dustpan and brush for cleanup - Equipment damage: Stop using equipment, report to teacher, do not attempt to fix

### Appendix D: Vocabulary List (English-Kiswahili)

| English Term | Kiswahili Translation | Definition |
| --- | --- | --- |
| Elasticity | Unyumbufu | Ability to return to original shape after deformation |
| Plasticity | Ulaini | Permanent deformation without breaking |
| Ductility | Uwezekano wa kusongwa | Ability to be drawn into wires |
| Malleability | Uwezekano wa kufuliwa | Ability to be hammered into sheets |
| Brittleness | Udhaifu / Uvunjavu | Tendency to break without significant deformation |
| Hardness | Ugumu | Resistance to scratching or indentation |
| Stiffness | Ukakamavu | Resistance to deformation |
| Strength | Nguvu | Ability to withstand applied forces |
| Toughness | Uthabiti | Ability to absorb energy before breaking |
| Force | Nguvu | Push or pull on an object |
| Extension | Ongezeko la urefu | Increase in length |
| Spring constant | Thabiti ya chemchemi | Measure of spring stiffness |
| Stress | Shinikizo | Force per unit area |
| Strain | Mnyogeo | Fractional change in length |
| Young’s modulus | Moduli ya Young | Measure of material stiffness |
| Elastic limit | Kikomo cha unyumbufu | Maximum stress before permanent deformation |
| Plastic deformation | Mnyogeo wa kudumu | Permanent change in shape |
| Breaking point | Hatua ya kuvunjika | Point at which material fractures |
| Cross-sectional area | Eneo la mkato | Area of a slice through an object |
| Proportional | Sawia | Relationship where one quantity increases as another increases |

### Appendix E: Common Misconceptions and How to Address Them

| Misconception | Why It’s Wrong | How to Address |
| --- | --- | --- |
| “Elastic means stretchy” | Elastic means returns to original shape, not necessarily stretchy (glass is elastic but not stretchy) | Show examples: glass is elastic but brittle, rubber is elastic and stretchy. Emphasize “returns to original shape” |
| “Stress and force are the same” | Stress accounts for area; same force on different areas produces different stress | Demonstrate with thick and thin wires under same force—different extensions |
| “Strain has units of meters” | Strain is a ratio (ΔL/L₀), so it’s dimensionless | Emphasize that strain is a fraction or percentage, not a length |
| “A stiff material is always strong” | Stiffness (Young’s modulus) and strength (breaking stress) are different properties | Show glass: high E (stiff) but low breaking stress (weak/brittle) |
| “Hooke’s Law applies to all materials at all times” | Hooke’s Law applies only within the elastic limit | Demonstrate over-stretching a spring—permanent deformation shows limit |
| “The spring constant depends on how much you stretch the spring” | Spring constant is a property of the spring, not dependent on extension | Show that k = F/x is constant for different extensions (within elastic limit) |
| “Young’s modulus and spring constant are the same” | Young’s modulus is a material property (intrinsic); spring constant depends on material and geometry | Explain: E is for materials, k is for specific springs/objects |
| “Plastic deformation means the material is made of plastic” | Plastic deformation means permanent deformation, not related to plastic material | Clarify terminology: “plastic” as adjective (permanent) vs. “plastic” as noun (material) |
| “Area under stress-strain curve is Young’s modulus” | Area represents energy absorbed (toughness); slope in elastic region is Young’s modulus | Annotate graph clearly: slope = E, area = toughness |
| “All metals are ductile” | Some metals are brittle (e.g., cast iron) | Provide counterexamples and explain that ductility varies among metals |

### Appendix F: Extension Activities and Projects

1. Design Challenge: Build a Shock Absorber - Task: Design and build a model shock absorber using springs, rubber bands, or other elastic materials. - Requirements: Must absorb impact from a dropped mass, protect a “payload” (e.g., egg), use materials available locally. - Assessment: Test effectiveness, explain material choices using mechanical properties, present design to class.

2. Research Project: Material Failures - Task: Research a real-world structural failure (bridge collapse, building failure, etc.) and analyze the role of material properties. - Requirements: Identify materials used, explain what went wrong (material failure, design flaw, overload), propose improvements. - Presentation: Create poster, slideshow, or video explaining findings.

3. Community Investigation: Local Construction Materials - Task: Survey construction sites or buildings in your community. Identify materials used and evaluate their suitability. - Requirements: Interview builders or engineers, photograph materials, analyze choices based on mechanical properties, cost, and availability. - Output: Report with recommendations for improving material selection in local construction.

4. Biomimicry Exploration: Nature’s Materials - Task: Research how natural materials (spider silk, bone, wood, shells) achieve remarkable mechanical properties. - Requirements: Explain structure-property relationships, compare to engineered materials, propose bio-inspired innovations. - Presentation: Create infographic or presentation for younger students.

5. Advanced Investigation: Temperature Effects - Task: Investigate how temperature affects mechanical properties (e.g., spring constant at different temperatures). - Requirements: Design experiment, collect data, analyze results, explain molecular basis. - Output: Lab report with graphs and analysis.

6. Material Database Creation - Task: Create a comprehensive database of materials available in Kenya with their mechanical properties, costs, and applications. - Requirements: Research multiple sources, organize data in spreadsheet or database, include images and descriptions. - Use: Share with class as reference resource.

7. Video Tutorial Creation - Task: Create educational videos explaining mechanical properties concepts for younger students or peers. - Requirements: Clear explanations, demonstrations, examples, accurate content. - Platform: Share on school website, YouTube, or in class.

8. Jua Kali Innovation Study - Task: Visit Jua Kali workshops and document how artisans select and work with materials. - Requirements: Interview artisans, observe techniques, analyze material choices, document innovations. - Output: Photo essay, video documentary, or written report.

### Appendix G: Digital Resources and Links

PhET Simulations (phet.colorado.edu): - “Hooke’s Law” - Interactive spring simulation - “Masses and Springs” - Explore spring behavior - “Elasticity” - Advanced material behavior simulation

Khan Academy (khanacademy.org): - “Introduction to Materials Science” - “Hooke’s Law Explained” - “Stress and Strain” - “Young’s Modulus” - “Understanding Stress-Strain Curves”

YouTube Channels: - Crash Course Physics - Material properties overview - The Efficient Engineer - Stress-strain graphs explained - Real Engineering - Material selection in engineering - Practical Engineering - Construction materials and failures

Interactive Tools: - Desmos Graphing Calculator (desmos.com) - For plotting force-extension and stress-strain graphs - Google Sheets / Excel - For data analysis and graphing - Material Property Databases: - MatWeb (matweb.com) - AZoM (azom.com)

Videos Specific to Kenya Context: - Search YouTube for: “Kenya construction,” “Jua Kali metalworking,” “matatu suspension” - Local news reports on building safety and material quality

Safety Resources: - OSHA Laboratory Safety Guidelines - Kenya Bureau of Standards (KEBS) material standards

### Appendix H: Assessment Rubrics

(Rubrics for Final Model, Written Assessment, and Practical Task are included in Section E above.)

Additional Rubric: Laboratory Investigation Participation

| Criterion | Excellent (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- |
| Preparation | Arrives prepared with materials, notebook, and prior knowledge reviewed | Arrives mostly prepared | Arrives somewhat unprepared | Arrives unprepared |
| Engagement | Actively participates in all aspects of investigation | Participates in most aspects | Participates minimally | Does not participate meaningfully |
| Collaboration | Works effectively with team, shares tasks, communicates clearly | Works adequately with team | Limited collaboration | Does not collaborate or disrupts team |
| Data Collection | Collects accurate, detailed data with proper units and labels | Collects mostly accurate data | Collects incomplete or inaccurate data | Does not collect data |
| Safety | Consistently follows all safety procedures | Mostly follows safety procedures | Occasionally violates safety procedures | Frequently violates safety procedures |
| Analysis | Thoroughly analyzes data, identifies patterns, draws conclusions | Adequately analyzes data | Limited analysis | No meaningful analysis |
| Reflection | Reflects deeply on results, identifies sources of error, proposes improvements | Reflects adequately | Limited reflection | No reflection |

### Appendix I: Exit Ticket Templates

Lesson 1 Exit Ticket: Define two material properties and give one example of each from real life.

Lesson 2 Exit Ticket: Explain Hooke’s Law in your own words. Give one real-world example where Hooke’s Law applies.

Lesson 3 Exit Ticket: Given: Force = 100 N, Area = 5 × 10⁻⁶ m², Original length = 1.5 m, Extension = 3 mm Calculate: a) Stress, b) Strain

Lesson 4 Exit Ticket: Explain what Young’s modulus is and why it’s useful for comparing materials. Give one example.

Lesson 5 Exit Ticket: Sketch a stress-strain graph and label: elastic region, elastic limit, plastic region, ultimate tensile strength, breaking point.

Lesson 6 Exit Ticket: (Reflection) What was most interesting in this unit? What was most challenging? How will you use this knowledge in the future?

### Appendix J: Parent/Guardian Communication Letter

Dear Parent/Guardian,

Your child is beginning a new unit in Grade 10 Physics: Mechanical Properties of Materials. This unit explores how different materials respond to forces—why some materials stretch and return to their original shape (like springs), while others deform permanently (like clay) or break (like glass).

What We’ll Learn: - Material properties: elasticity, plasticity, ductility, malleability, brittleness, hardness, stiffness, strength, toughness - Hooke’s Law: the relationship between force and extension in springs (F = kx) - Stress and strain: how to measure and compare material behavior - Young’s modulus: a measure of material stiffness - Stress-strain graphs: interpreting material behavior graphically - Applications: material selection for construction, transportation, and manufacturing in Kenya

How We’ll Learn: - Hands-on investigations with springs, wires, and various materials - Predict-Observe-Explain (POE) cycles to develop scientific thinking - Building and revising explanatory models - Think-Pair-Share discussions and collaborative group work - Digital simulations and videos - Real-world applications to Kenya contexts (construction, matatus, Jua Kali sector)

How You Can Support: - Ask your child to explain what they’re learning about materials and forces - Point out examples of different materials in everyday life (buildings, vehicles, household items) - Discuss why certain materials are chosen for specific purposes - Encourage curiosity and questioning about the physical world - Ensure your child completes homework and brings necessary materials to class - Support safe practices if your child conducts investigations at home

Assessment: Your child will be assessed through: - Ongoing formative assessment (models, investigations, discussions, exit tickets) - Final explanatory model answering the unit’s driving question - Written assessment (multiple choice, calculations, graph interpretation, short answer) - Practical task (material selection and justification) - Participation and process throughout the unit

Safety: All laboratory investigations will be conducted with appropriate safety measures. Your child will wear safety goggles and follow safety procedures. Please discuss the importance of laboratory safety at home.

Questions or Concerns: If you have any questions about this unit or your child’s progress, please contact me at [teacher contact information].

Thank you for your support in your child’s science education!

Sincerely,
[Teacher Name]
Grade 10 Physics Teacher

### Appendix K: Teacher Reflection Prompts

After Each Lesson: 1. Did learners achieve the learning outcomes for this lesson? 2. What evidence do I have of learner understanding? 3. What misconceptions emerged, and how did I address them? 4. Were the activities appropriately paced and engaging? 5. Did all learners have access to the learning (differentiation effectiveness)? 6. What adjustments do I need to make for the next lesson?

After the Unit: 1. Overall Effectiveness: - Did learners answer the driving question successfully? - What percentage of learners met, exceeded, or fell below expectations on the summative assessment? - What evidence shows growth from initial to final models?

- Instructional Strategies:

  - Which CBE strategies (POE, DQB, Think-Pair-Share, model building) were most effective?

  - How well did hands-on investigations support learning?

  - Were digital resources accessible and useful?

  - Did Kenya-specific contexts enhance engagement and relevance?

- Challenges:

  - What challenges did learners face (conceptual, mathematical, language, practical)?

  - What challenges did I face as a teacher (time, resources, classroom management)?

  - How did I address these challenges?

- Differentiation:

  - Did differentiation strategies effectively support struggling learners?

  - Were advanced learners sufficiently challenged and extended?

  - Did ELL supports enable full participation?

- Assessment:

  - Were assessment tasks aligned with learning outcomes?

  - Did rubrics clearly communicate expectations?

  - Was feedback timely and actionable?

  - Did learners have opportunities to revise and improve?

- Improvements for Next Time:

  - What worked well and should be kept?

  - What needs revision or replacement?

  - What additional resources or support do I need?

  - How can I better integrate core competencies, values, and PCIs?

- Professional Growth:

  - What did I learn about teaching this content?

  - What new skills or knowledge do I need to develop?

  - How can I share successes and learn from colleagues?

### Appendix L: Teacher Preparation Checklist

Before the Unit: - [ ] Read through entire lesson sequence and understand learning progression - [ ] Gather all materials: springs, masses, wires, rulers, vernier calipers, safety goggles, various material samples - [ ] Test all equipment to ensure it works properly - [ ] Preview all digital resources (PhET simulations, Khan Academy videos, YouTube videos) and ensure they are accessible - [ ] Prepare Driving Question Board (poster or digital display) - [ ] Set up laboratory/classroom space for hands-on investigations - [ ] Review safety procedures and ensure safety equipment (goggles, first aid kit) is available - [ ] Prepare worksheets, handouts, templates (POE, data tables, graphs, model templates, exit tickets) - [ ] Create or print assessment materials (written test, practical task, rubrics) - [ ] Communicate with parents/guardians (send letter home) - [ ] Arrange for any guest speakers or field trips (optional: visit construction site, Jua Kali workshop) - [ ] Prepare differentiation materials (vocabulary cards, formula sheets, simplified/extended tasks)

During the Unit: - [ ] Display and maintain Driving Question Board throughout unit, updating after each lesson - [ ] Collect and review models after each revision to track learner progress - [ ] Use formative assessment data to adjust pacing and instruction - [ ] Emphasize real-world connections and Kenya-specific contexts in every lesson - [ ] Foster collaborative culture through consistent use of Think-Pair-Share and group work - [ ] Provide timely, specific feedback on investigations, calculations, models, and exit tickets - [ ] Differentiate based on ongoing assessment of learner needs - [ ] Monitor safety during all laboratory activities - [ ] Celebrate successes and learning milestones (e.g., “We can now calculate stress and strain!”) - [ ] Address misconceptions as they arise using evidence and discussion - [ ] Document learner work (photos, samples) for reflection and sharing

After the Unit: - [ ] Administer all summative assessment components (final model, written test, practical task) - [ ] Grade assessments using provided rubrics within one week - [ ] Provide feedback to learners on their performance (written comments, conferences) - [ ] Analyze assessment results to identify strengths and areas for improvement (class-wide and individual) - [ ] Reflect on unit effectiveness using teacher reflection prompts (Appendix K) - [ ] Gather learner feedback (survey, discussion) on what worked well and what could be improved - [ ] Document successes, challenges, and lessons learned for future reference - [ ] Revise and refine lessons based on reflection and feedback - [ ] Share successes and challenges with colleagues (department meetings, professional learning communities) - [ ] Communicate results to parents/guardians (report cards, parent-teacher conferences) - [ ] Plan for remediation (small group instruction, additional practice) or extension (independent projects) as needed - [ ] Archive materials and resources for future use - [ ] Celebrate with learners (display final models, share applications, recognize growth)

## FINAL NOTES

This revised CBE lesson sequence for Grade 10 Physics: Mechanical Properties of Materials now STRICTLY follows the established template format with:

✅ Section A: Comprehensive sub-strand overview
✅ Section B: 6 detailed lesson plans (12 periods total)
✅ Section C: 5-COLUMN SUMMARY TABLE with minute-by-minute learner experience for all lessons: - Learner Experience - Resource Link - Teacher Moves - Sensemaking Strategy - Formative Assessment
✅ Section D: Extensive differentiation strategies
✅ Section E: Comprehensive final synthesis and assessment
✅ Appendices: All supporting materials (formulas, reference tables, safety guidelines, vocabulary, misconceptions, extensions, resources, rubrics, exit tickets, parent letter, teacher checklist)

The unit addresses the phenomenon of materials responding differently to forces, answers the driving question through systematic investigation, covers all required content (material properties, Hooke’s Law, stress, strain, Young’s modulus, stress-strain graphs, applications), employs CBE strategies (POE, DQB, model building/revision, Think-Pair-Share), integrates digital resources (PhET, Khan Academy, YouTube), emphasizes Kenya contexts (construction, matatus, Jua Kali), includes hands-on investigations, develops core competencies, integrates values, addresses PCIs, and provides comprehensive assessment.

This document is ready for implementation and provides everything a teacher needs to deliver high-quality, competency-based instruction on mechanical properties of materials.

END OF REVISED DOCUMENT